# Case Studies in Next Generation Manufacturing

Or

# Case Studies in Integrated and Continuous Processing

Or

# Case studies in the evolution of batch to continuous bioprocessing

Many are considering converting fed-batch processes to next generation integrated and continuous bioprocesses (ICB) and are challenged with applying existing platform knowledge, matching product quality to fed batch, and using existing equipment and facilities.

We describe the adaptation of our batch upstream and downstream mAb platform to ICB at the development and clinical scale.  We adapted four existing cell lines, fed-batch media, and fed-batch bioreactors to a continuous perfusion process with a partially integrated downstream that resulted in between three and four times more productivity per bioreactor volume. The perfusion media was a combination of the fed-batch basal media and the feed media, with some unneeded components removed to enable concentration. The bioreactors were adapted by the addition of an alternating tangential flow filtration unit (ATF) and larger mass-flow controllers. The bioreactors were characterizated in regimes not previously needed for fed batch to allow the needed mixing, O2 supply, CO2 stripping, and heat removal to support cell densities in excess of 150M/cells/mL. In general, there was no need to change the impellers or spargers to adapt to the high density. The bioreactor runs were developed at the 3L scale, and scaled up to the 100L, 200L, and 500L scale depending upon the material need.

The partially integrated downstream processes were based on the existing batch processes. The downstream used single column chromatography with largely the same buffer conditions and resins as the batch process. The lot was pooled after the Protein A step, and processed downstream as a single lot. The lot size was severely limited by the downstream buffer volume, which in turn was limited by floorspace in the clinical manufacturing suites. [Buffer concentrates were developed to partially overcome this limitation.] The process used automation similar to the fed-batch process, with concomitant increase in manual operation due to the number of chromatographic cycles required. While a higher degree of automation and multicolumn chromatographic steps would have benefited the downstream operation, the use of existing systems facilitated the adoption of ICB without precluding the use of a more optimal downstream in late-stage and commercial manufacturing. The material generated was [hopefully] comparable to the fed-batch material. This strategy allows the use of existing facilities to make material representative of future fully integrated and continuous bioprocessing equipment.