# A Framework for Integrated and Continuous Processing

© 2020 AstraZeneca

# Abstract

There is a growing interest in using continuous and integrated bioprocessing (ICB) as a manufacturing platform for the production of recombinant protein therapeutics produced from mammalian cells.

At first glance, the newly evolved ICB has created a vast diversity of platforms. A closer inspection reveals convergent evolution: nearly all of the major ICB methods have a common denominator framework that could allow manufacturing across a global ecosystem of manufacturers using simple, yet effective, equipment designs. This simple framework is not as efficient or productive as the many sub-species of ICB promoted by those active in this space. The framework is, however, capable of supporting the manufacturing of most major biopharmaceutical ICB without major changes in the regulator licensure, while achieving most of the efficiency and productivity benefits of the more varied ICB platforms.

The common framework lends itself to implementation in a stepping-stone approach. The biggest change, and biggest benefit, comes from moving from fed-batch cell culture to high-density, continuous perfusion cell culture, linked with an integrated batch downstream. Manufacturers can first implement single-column (SCC) then multicolumn capture (MCC)  capture in stages throughout the lifecycle of a project. The framework also supports legacy fed-batch processes.

This paper reviews the ICB that are being used, or are soon to be used, in a GMP manufacturing setting for recombinant protein production. The ICB platforms are used primarily for mAb and mAb-like products, yet can be adapted to all recombinant proteins. This paper provides examples of the design basis, including flow rates, column sizes, material production, buffer and media volumes, for a plant capable of manufacturing as little as 6kg to as much as 200kg per batch or up to 4 tons per year.

# Introduction

Many biopharmaceutical companies are considering switching from fed-batch bioreactors and independent, batch downstream unit operations to integrated and continuous bioprocessing (ICB), which uses continuous perfusion bioreactors and a linked and somewhat continuous downstream. ICB has well-reviewed advantages in the cost of clinical [ref] and, for some scales, commercial [ref] manufacturing. The productivity of capital (kg production/$ invested) can be higher for ICB, because ICB equipment operates 24 hours per day, nearly 365 days per year. Due largely to automation, ICB can have increased labor productivity [ref]. ICB has also been shown to have similar or worse operating costs compared to fed-batch for certain situations [ref]. The largest benefit to ICB appears to be when perfusion bioreactor with a productivity five-fold over that of fed batch is linked to an automated, fully utilized downstream.

While there is significant synergy between ICB and single-use manufacturing, they should not be conflated. While most of the benefits of ICB can be obtained in either stainless-steel plants or single-use plants, the productivity of perfusion bioreactors enables single use bioreactors (SUB) to be as productive as plants based on larger and more expensive stainless-steel bioreactors [ref]. SU technologies and ICB increase the productivity of capital by reducing the overall capital cost. Due to shorter build times, SU plants also allow capital investment with less risk [ref], which also increases the productivity of capital. Most authors agree that the bioreactor is the most important operation to convert to single use due to its simplicity, fast change over, not needing clean steam utilities, low capital cost, and low labor requirement. The authors largely agree that downstream unit operations can be either stainless steel or single use.

Similarly, many of the benefits attributed to ICB are due to high degree of automation, which reduces labor costs. This same degree of automation would also reduce labor costs for classic batch processing as well.

Here we report and review several of the largest biopharmaceutical companies ICB platforms to determine if there exists a common denominator in the industry. These processes are not often well described in the literature, and a significant amount of information comes from public presentations. There are many ICB processes that have not found their way into common practice and will not be discussed. Many of these processes are valuable and will likely be implemented in the future. This article will not disclose individual companies’ platforms except where a citation exists, or disclosure permission is given. Other features will remain proprietary and will not be described.

After discussing the various ICB implementation strategies, we outline a stepping-stone approach for conversion from fed-batch to perfusion bioreactors with an integrated downstream. This will allow the use of existing manufacturing facilities to use ICB and obtain most of the benefits associated with ICB.

We then conclude with a quantitative description of the framework integrated and continuous bioprocess. This process description is sufficient for companies to develop an ICB platform, and as a design basis for equipment vendors and CMOs. Some features that companies have built on top of the common denominator will be described in sufficient detail to allow CMOs and vendors to design equipment that will allow expansion.

At a high level, the integrated and continuous bioprocesses being used by industry reflect the mAb platform processes [Kelley 2008, etc] upon which they were built. These processes begin with bioreactors with only minor modifications from those used for decades. They link the bioreactor directly to an integrated downstream that is based on batch chromatography processes that also have been used for decades. In many cases, the chemistry (resins, buffer composition, and membrane composition) is identical to historical platform. Product pools are eliminated or minimized. Media and buffer concentrates are required due to space constraints for the more productive processes. In-line titration is also required for many linked unit operations. Process Analytical Technologies (PAT) are used, but in many cases, they are not much more complex than those used in batch processes.

N-1 Bioreactor description

The N-1 bioreactor is often the first step that differs from classical batch processing. This step is often intensified by adding a perfusion step to achieve a high density of viable cells, which increase the inoculation cell density for the N-stage reactor. Like many of the process intensification methods described here, the N-1 perfusion step can be used in batch processing and in ICB. Indeed, N-1 perfusion has been used for decades at the 2000L scale [Ref Enbrel, etc]. These processes used tangential flow filtration (TFF) with flat-sheet membranes as a cell-retention method. More recently, many processes use alternating tangential flow filtration (ATF) with hollow fibers [ref Repligen, etc]. ATF uses alternating retentate flow to reverse the Starling flow in the hollow fiber, which allows cleaning of fouled membrane, and uses more of the membrane surface area than standard TFF [Ref Flaka Radoniqi 2018]. These attributes of ATF are more for the N-stage perfusion bioreactor.

Bioreactor description (FB and perfusion)

Many companies have moved to, or are exploring, perfusion-based bioreactor processes. Several have developed high-intensity fed-batch processes. The high-density cultures require higher gas transfer, rich, concentrated media, and PAT that supports media feed strategies. The bioreactor designs discussed later will support both perfusion-based and high-intensity fed-batch processes. Some authors believe that the batch process will give way to the perfusion processes as companies continue to develop perfusion processes that exceed 5x the productivity of the batch bioreactors.

- Why perfusion

- SU and SS

- Number of bioreactors

  - Run in staggered with staged downstream

  - Run staggered with same downstream

Cell Retention device

- ATF—typically up to 500L or 1000L

- TFF—more likely at 1000L or above

- Other methods (short paragraph listing inclined settler, spin filters,

Protein A Capture step

- Pooling before VI option with stability discussion

- Surge tank

Low pH virus inactivation

- Pooling after VI option

Polishing Step 1, typically AEX

- Pooling after AEX option with stability discussion

Polishing Step 2, typically CEX

- Pooling after CEX option

VRF

UFDF or SPTFF and ILDF

- End-to-end processing option

- Impact of product quality variability during run

- High level NGM framework

  - Review of publicly disclosed IBC (see table and figure)

# Assumptions

The assumptions in the framework process are described below. These assumptions are typical throughout the industry and do not represent confidential information.

| Bioreactor |  |
| --- | --- |
| Bioreactor Size (L) | 500-2000 |
| Avg cell density (million cells/mL) | 50-150 |
| Qp (pg/cell/day) | 20-60 |
| Permeate Perfusion Rate (bioreactor volumes/day, vvd) | 1.5 |
| Batch Duration | 14-30 |
| Media Concentration Factor | 1-5x |

| General Column Assumptions |  |
| --- | --- |
| Linear flowrate (cm/hr) | 300 |
| Bed Height (cm) | 10-20 |
| Buffer Concentration Factor | 5 |
| # Columns | 1-2 |

| Chromatography | Protein A | Flow Through | Bind/Elute |
| --- | --- | --- | --- |
| Capacity (g/L) | 30-60 | 50-150 | 30-60 |
| Buffer Volume (1x) used (Column Volume, CV) | 22 | 16 | 16 |
| Elution Volume collected (CV) | 1.5-3 | 0.2-0.5 | 1.0-1.5 |
| Yield | 90% | 95% | 85% |

| Virus Filtration |  |
| --- | --- |
| Capacity (L/m2) | 700-1400 |
| Flux (LMH) | 100 |
| Yield | 95% |

| UFDF |  |
| --- | --- |
| Membrane Capacity (g/m2) | 800-1500 |
| Yield | 95% |
| Cross Flow (LMH) | 300-500 |
| Concentration 1 Flux (LMH) | 30-60 |
| Concentration 1 target (g/L) | 50 |
| # Diavolumes | 6 |
| Final DS Conc Target (g/L) | 100 |