# A Common Framework for Integrated and Continuous Processing

© 2020 AstraZeneca

Authors

Jonathan Coffman,  Mark Brower, Lisa Connell-Crowley, Sevda Deldari, Suzy Farid, Ujwal Patil, , David Pollard, Maen Qadan, Steve Rose, Eugene Schaefer, Joseph Shultz

# Abstract

There is a growing application of integrated and continuous bioprocessing (ICB) as a manufacturing platform for the production of recombinant protein therapeutics produced from mammalian cells. At first glance, the newly evolved ICB has created a vast diversity of platforms. A closer inspection reveals convergent evolution: nearly all of the major ICB methods have a common framework that could allow manufacturing across a global ecosystem of manufacturers using simple, yet effective, equipment designs. The framework is capable of supporting the manufacturing of most major biopharmaceutical ICB and legacy processes without major changes in the regulatory license.

This paper reviews the ICB that are being used, or are soon to be used, in a GMP manufacturing setting for recombinant protein production from mammalian cells. The adaptation of the various ICB modes to the common ICB framework will be discussed, along with the pros and cons of such adaptation. The equipment used in the common framework is generally described.

An example plant built on the common framework will be discussed. The flexibility of the plant is demonstrated with batches as small as 6kg or as large as 500kg. The yearly output of the plant is as much as 8 tons.

# Introduction

Many biopharmaceutical companies are considering switching from fed-batch bioreactors and independent, batch downstream unit operations to integrated and continuous bioprocessing (ICB), which uses a mammalian host in continuous perfusion bioreactors and a linked and continuous downstream. ICB has well-reviewed cost advantages for clinical and, for some scenarios, commercial manufacturing (Arnold et al., 2019; Hummel et al., 2019; Klutz et al., 2016a; Levine et al., 2013; Pollock et al., 2013a; Pollock et al., 2013c; Walther et al., 2015).

The literature describes a dizzying array of options for ICB, so much so that one might think that biopharmaceutical companies would choose different and incompatible implementations.  Here we report and review many of the largest biopharmaceutical companies implementing ICB to show there exists a common denominator in the industry. We further demonstrate that the common framework uses many of the familiar batch processing operations, and allows backward compatibility with legacy batch processes.

The specific choices made for implementing ICB in GMP manufacturing are not often well described in the literature. Only a fraction of published and presented work is implemented. The common framework process is based on the authors’ knowledge of what is being implemented in their company, as well as interviews with other bioprocess engineers across the industry. This article will not disclose the details of individual companies’ platforms except where a citation exists, or disclosure permission is given.  Some features that build on top of the framework will remain proprietary and will not be described.  All of the integrated and continuous bioprocesses will fit into a plant built on the common framework without significant regulatory changes and without large capital investments. The framework will not require new process characterization studies, nor will it require new virus removal or inactivation studies. Regulatory filings and technology transfer would, however, benefit from some foreknowledge of this framework process.

These considerations are important, especially today. Due to the global COVID-19 pandemic, many companies have reserved space in CMOs with >12,000L bioreactor capacity to make clinical and commercial material for possible antibody therapies (Kelley, 2020), leading to capacity limitations for fed-batch processes. These plants cannot quickly be expanded. Stainless-steel plants take 5 years or more to design and build. Smaller single use plants can be built in as little as 12 months, but these smaller plants simply will not have the capacity or the cost structure to make sufficient material to prevent and treat COVID 19 if they use conventional batch operations. Using the common framework for ICB described in this article would allow quickly built-single use facilities to produce as much as 8 tons of material per year.

We report in a companion article in this issue [Coffman 2020] the design basis for the framework integrated and continuous bioprocess which is sufficiently detailed to allow CMOs and bioprocessing vendors to design plants and equipment that would enable a global ecosystem high productivity manufacturing sufficient to supply mAbs for treating and preventing COVID19 and other diseases. This framework ICB is being used as the basis for the “first generation” of ICB manufacturing for NIIMBL, a US government/academic/industrial bioprocessing consortium, also co-published in this issue [Erikson et al 2020].

# The Common Framework for Integrated and Continuous Manufacturing

The known ICB being implemented in clinical or commercial manufacturing are shown in Table 1 (Brower et al., 2020). These processes use a mixture of N-stage perfusion, and some form of multicolumn chromatography with linked processes.  Some processes are integrated end-to-end, from the bioreactor though to the final concentration step. Others pool the product in-process to define a lot.  Some processes run for 14 days, many run for 20-30 days. Some use single-column chromatography; most use some form of multicolumn chromatography. While it may seem that a single set of equipment cannot enable these different permutations on ICB, a common framework has been designed.

The common ICB framework that fits these processes is shown in Figure 1. The common framework is possible because each ICB was derived from a common batch mAb platform processes for monoclonal antibodies (Kelley, 2009).

The bioreactor in the ICB framework is run continuously, harvesting the product through a filtration step. The bioreactor links to an integrated downstream that is based on batch chromatography processes. Instead of simulated moving bed chromatography, the framework uses dual column chromatography, which is compatible with single column chromatography. In many cases, the chemistry (resins, buffer composition, and membrane composition) is identical to the historical platform. Product pools are replaced with smaller surge tanks. The low pH virus inactivation step is flexible, allowing batch or continuous inactivation. The common framework uses media and solution concentrates, more preferably, or in-line conditioning to enable large batch sizes. In-line titration is also required for many linked unit operations. Process Analytical Technologies (PAT) are used, but in this common framework, they are not more complex than those used in batch processes. The common framework also supports fed-batch bioreactor processes with a filtration harvest step.

# N-stage Bioreactor

The framework uses perfusion bioreactors to achieve high specific productivity in either a steady state or dynamic perfusion (Wolf et al., 2020).  The industry has many years of commercial perfusion manufacturing scale experience at large scale up to 4000 L (Pollock et al., 2013b).   Over 17 commercially launched biologics have used perfusion over the last 25 years using a range of cell retention devices including gravitational settlers, ATF and TFF (Clincke et al., 2013). The industry has seen cell specific productivities in the range of 20 to 90 pg/cell/day range (Alldread et al., 2014; Li et al., 2010) with cell densities in the 30-70 Million cells/mL. Typically constant cell-density perfusions has been  achieved at 2- -4 g/L day at 70 M cells/mL for up to 60-80 days (Brower et al., 2015; Warikoo et al., 2012) [Warikoo], [Ozturk]. Dynamic perfusion, which allow variable cell densities, has seen cell densities between 100 and 200 M cells/mL. Given this industry experience and that this framework is designed to be compatible with the future productivities, a relatively high bioreactor productivity of 4.8 g/L/day (Tables 2 and 3) was chosen. This assumes the relatively high cell specific productivity, 40pg/cell/day and a high average cell density of 120Mcells/mL which has been achieved at lab scale. We expect this productivity to be routine during the next five years.  Indeed, as discussed below, the only upper limit to the framework process with respect to bioreactor productivity is solution preparation and storage, and the maximum flow rate on the Protein A skid.

We have found that cell lines that have been selected for fed batch adapt easily to perfusion cell culture. The amount of media required for perfusion bioreactors is many-fold higher than that of fed-batch. Perfusion media must be developed that minimizes the volume of media required. Perfusion media can be adapted directly from fed-batch media by mixing a portion of the basal media with the concentrated feed media, removing components unneeded for perfusion, and separating acid, base, and neutral soluble components (Dennis M. DiSorbo, 1995). This media can be from three to five-fold more concentrated than the original fed-batch media. The example below assumes that three-fold media concentrates are used. Expensive media components such as insulin, di-peptides, or hydrolysates should be avoided.

Some ICB will not have three-fold media concentrates. The framework process would require either more frequent media preparation during the batch or the use of fewer bioreactors.

# Cell Retention

Cells are retained in the bioreactor with tangential flow filtration, operated in one of two modes: alternating tangential flow filtration (ATF) system, or tangential flow filtration (TFF).  This approach has enabled high cell concentrations up to 100 million cells /mL and established up to 2000L scale.  Nevertheless the cell densities increased the fouling of filtration membranes continues to be a concern (Pinto et al., 2020; Wang et al., 2019). These can lead to shortening of manufacturing campaigns (van Reis and Zydney, 2007) and to the routine cumbersome switching of filtration systems during manufacturing operations.  The fundamental mechanistic understanding of membrane fouling with undesired product retention (product sieving) and eventually membrane plugging remains poorly understood. Mechanistic studies propose biological material deposits onto the microfiltration membrane forming a cake that blocks the pores (Kelly et al., 2014) causes product sieving (Bolton and Apostolidis, 2017; Wang et al., 2019). Performance comparison of ATF to TFF show a lower sieving decay for ATF, potentially due to the bidirectional flow, minimizing the biomaterial deposit on the hollow fiber membrane (Radoniqi et al., 2018; Wang et al., 2017). However the ATF has been associated with operational instability at higher cell densities (Pinto and Brower, 2020; Pinto et al., 2020).    Backflushing of the ATF and TFF filters with perfusate or fresh media has shown potential to extend filtration life during a manufacturing campaign (Wang et al., 2017; Wang et al., 2019). Researchers have evaluated different membranes chemistries and pore size structures as opportunities to reduce product sieving that typically can reach as low as 50-60% transmission after only 20 days of operation. Recently the product sieving & fouling has been successfully mitigated to allow > 85% transmission for 40 days by the use of wide pore filters. (Pinto et al., 2020; Wang et al., 2019). Despite these improvements, it is clear that opportunities remain to improve the understanding of fouling and the potential for pursuit of non membrane approaches (Bettinardi et al., 2020; Kwon et al., 2017).

ATF systems are often used for 500L bioreactors and below; Two or more ATF systems may be used to support a 1000L perfusion bioreactor, but TFF is the preferred choice. Some authors have developed processes that use as many as four ATFs to support a 2000L perfusion bioreactor. They take up considerably more floor space than the TFF systems, and must be immediately adjacent to the bioreactor. ATFs require less development time than TFF systems, and thus may be used in early phase clinical trials or commercial products that do not have high material needs. Many, but not all, of the authors agree that TFF systems are preferred.

The common framework would use the same permeate pumps for both the ATF and TFF set up.  The retentate pumps and associated controllers would be unique to either the ATF or TFF set up.

# Dual-column chromatography

The chromatography steps are the most variable from company to company. Many companies use three or more columns (Zydney, 2016). Some use only one. Some use a different numbers depending upon the step. The simplest common framework for these various chromatography processes is two columns. Although many processes use more than two columns, the multicolumn nature is not used in any step but the load step: The systems use two columns during the load step to overload one column while capturing the product in the flow through onto the second column (Angarita et al., 2015).  All other steps are effectively single-column operations. Thus, the simplest embodiment of the multicolumn chromatography skid is a two-column system. A two-column system has about 1/4 fewer valves and ½ the number of pumps as a three-column system. In addition, the continuous operation of the dual-column process reduces the Protein A column volume from about 1500L for a 10 ton/year plant (Kelley, 2007) to two 50L columns. The difference in cost is $0.8M vs $23M. This 100L reaches its lifetime in 200 cycles, or at the end of every 500kg batch; the 1500L reaches its lifetime in 200 cycles in about 1.5 years if the plant makes only one product. Many plants make 3-8 products, meaning that the 1500L may reach its expiry (typically 5 years) prior to reaching its lifetime. Further, the simplicity of dual-column chromatography keeps the system cost low, and allows operators familiar with batch chromatography to easily operate the system.

Rather than use a dual-column system, it may be possible to link two standard single-column systems side-by-side. Many single-column systems are less than half the cost of a multicolumn system. Each single column system could, in addition, be used separately for more conventional processes, increasing the flexibility of the plant.  Automation allowing the tandem operation of two single-column skids is not a trivial task.

The dual-column system can be used as a replacement for one or more columns without impacting the regulatory process description. More columns do, however, increase the apparent continuity of the product flow into and out of the step. Also, above a certain titer, the two-column process cannot be continuous because the load step is completed faster than the subsequent wash, elution, regeneration steps. A dual-column process therefore requires a surge tank for the load when the titer exceeds a threshold.

In addition to protein A chromatography, the common framework employs one or two polishing chromatography operations to remove aggregates, host cell protein, and DNA and provide some viral clearance. Flowthrough chromatography is preferred because it is easy to integrate with other steps, requires fewer buffers and uses less solution volume (Table 2)  than bind/elute chromatography, and allows for a more continuous product flow through the process (Brown et al., 2010; Ichihara et al., 2019; Ichihara et al., 2018; Liu et al., 2011; Reck et al., 2017; Vogg et al., 2020).  Typical flowthrough polishing steps include AEX or mixed mode resins with loadings of >100 g/L resin that capture impurities and aggregates as the product flows through. Another option is frontal CEX chromatography, in which the product binds to the resin but is displaced by aggregates and impurities that bind more tightly (Reck et al., 2017; Vogg et al., 2020). This format has achieved loadings exceeding 1000g/L resin. The polishing steps can be performed with either a dual column system or a single system with a surge tank upstream that allows for load to be collected while the column periodically cycles through the non load steps. Load conditioning occurs inline via pH titration, salt solution addition or WFI dilution, depending on the requirements of the step.

In-process pool stability is a key consideration for integrated processes. The process should be integrated and continuous at least up until a stable process intermediate is obtained. At this stable intermediate, a pool of the entire bioreactor run can be made and held. This pool can be defined as the lot.

The hold time surrounding this pool may be the largest regulatory consideration for the framework process. Unless the process was designed with such a hold in mind, in-process hold studies will be required. These studies are not difficult, unless an instability is found that affects product quality. In this case, the lot pool should be made in a more stable part of the process.

The hold in the surge tank is also a consideration. The surge tank may have a volume of about 2-4 hours of product. This hold time would require hold studies out to three or four time constants, or perhaps 12-16 hours. Less stable products would require smaller surge tanks commensurate with the validated hold time.

The solution and water utilization of the downstream is a major consideration, and in many cases the main bottle neck in the ICB downstream. The standard productivity measure used in the literature for this step, mass product per volume resin per unit time (Budzinski et al., 2019; Cataldo et al., 2020), is not the most relevant to manufacturing because the resin is almost never the limiting factor. Instead, one should consider a product mass index, or the solution mass per mass of product produced. This measure is the nearly same as the measure used to compare the environmental efficiency of processes, the Product Mass Index (PMI) , which would include all raw materials and consumables in the numerator.

The use of solution concentrates is essential to maximizing the productivity of ICB. Floor space for production would be mostly solution preparation and solution storage without solution concentrates. Even with five-fold concentrates, many authors have seen that the solution preparation and storge area is larger than the main production area. The volume and number of solutions drives the size, cost, complexity, and throughput of high-capacity ICB manufacturing facilities.

In-line conditioning (ILC) may provide a better solution management option, providing over 20x concentration factor for many solutions (Carredano et al., 2018). ILC does not simply dilute a solution with water, but mixes acid, base, concentrated untitrated buffer, and salt with water to achieve the correct solution composition. ILC has the capacity to impact manufacturing more than multicolumn chromatography in terms of solution management reduction. ILC systems could be attached directly to each chromatography step, which might be expensive. A full cost analysis for the common framework process has not been done.  One ILC system could supply concentrated downstream solutions that are then diluted by the skids, as above. Since the process uses over 14 different solutions, this method of solution management uses nearly the same amount of floor space as the in-line dilution alone.

ILC works best, and allows the best pH control, where the pH is near the pKa of the buffer. Concnetration fators higher than 10x will require the correct buffer choice.

These considerations also benefit classic batch processing, though not to the same degree.

# Low pH virus inactivation

The low pH inactivation step typically follows the Protein A step. The step can be operated in either periodic batch mode or with a continuous plug flow reactor. The framework process can satisfy both modes with in-line titration of the product stream. For typical product concentrations, the acid solution flow rate is proportional to the product concentration.

For the systems that use continuous inactivation, a plug-flow reactor (PFR) is required. Many PFG designs have been published for low pH virus inactivation (Gillespie et al., 2019; Klutz et al., 2016b; Martins et al., 2020; Parker et al., 2018). Since the amount of acid and base changes with product concentration, the stream might require dilution with water to maintain constant ionic strength. The subsequent chromatography step must be developed accounting for variable load flow rates.

Better pH measurements would be beneficial for this step. pH probes that do not require daily calibration, or can be calibrated automatically in closed systems, would be beneficial. pH probes that can withstand gamma irradiation would also be beneficial.

# Virus Reduction Filter

Earlier investigations about mechanistic understanding of virus filtration have reported the impact of factors such as variability in product concentration, pH, ionic strength of the feed and fluctuations in operating pressure (Bakhshayeshi and Zydney, 2008; Bohonak and Zydney, 2005; Bolton et al., 2005; Ireland et al., 2005; Rathore et al., 2014; Strauss et al., 2017; Syedain et al., 2006; Tateishi et al., 2001; Wickramasinghe et al., 2010). A complex interplay between these factors can impact the overall performance of the VRF process by eventual flux decay and accidental virus breakthrough in the filtrate.  Some processes are able to avoid this flux decay and operate essentially continuously for long periods, enabling end-to-end processing.  Other processes require operation for less than a day, essentially in batch-wise operation.

The filter capacity is often based on the volume passing through the filter. Therefore, the filter area used is the same independent of where the lot is pooled. The flow rates for each mode are quite different. The continuous end-to-end process has about 1/20th to 1/60th the flow rate of the entire lot of material at once on the VRF. The framework process pools material before the VRF for small lot sizes (such as those needed in early phase clinical trials), or changes the virus filter daily for large material requirements. A single use skid with a high-flow and low-flow tubing set and pumps would allow the processing of both modes of VRF.

# Concentration and diafiltration Steps

Many ICB use the same ultrafiltration and diafiltration (UFDF) set up as for batch. Processes that diafilter the entire lot of material at once will suffer from the need for very large pumps and filters. This strategy is not recommended.

The framework process can be configured to allow batch-wise UFDF or single-pass, continuous UFDF. The step uses the same pumps and similar filter areas, while the flow path is redefined to allow both modes of operation.

The batch-wise process is shown in Figure 5A. First, a small UF system concentrates the VRF stream to a desired level. Periodically, the concentrated material is moved to a small diafiltration system. After diafiltration, the sub-lot is pooled with other sub-lots. The larger pool is concentrated with a small UF system over the course of the run, allowing fine control of the final pool concentration.

The single-pass, continuous UFDF process uses SPTFF (Jungbauer, 2013). to continuously concentrate the product. The product then passes through an in-line diafiltration (ILDF) step made up of several stages of dilution and SPTFF modules. These modules are most efficient when operated in a counter-current mode (Arnold et al., 2019; Huter and Strube, 2019; Jabra et al., 2019; Nambiar et al., 2018; Rucker-Pezzini et al., 201). The final drug substance concentration is achieved through a final SPTFF step. A schematic of such a process is shown in Figure 5B.  Using counter current DF can reduce the diafiltration solution volume by roughly a factor of two, mostly by allowing a higher concentration of product during the diafiltration operation.

The membrane sizes for the batch-wise and continuous modes of operation are similar, on the order of 1m2. Smaller lots are accommodated by doing fewer sub-batches.

Material is pooled during the entire lot. As with fed-batch processes, the product quality from the bioreactor may vary with time. The control of the product quality requires a sophisticated GMP and regulatory strategy.

# Implementation of clinical and a commercial 8-ton per year ICB

A large-scale staged implementation of the framework is shown in Figure 4. The plant is designed with flexibility. It is constrained by two factors: solution and media preparation volume and the flow rate of the capture step. The total amount of media and/or solution that can be prepared was arbitrarily set to 250,000L, and the maximum flow rate of the Protein A step was set to 12.5L/min. The details around these assumptions are published in the accompanying article.

Stage one of the implementation uses a 500L SUB. It is capable of producing as small as a 0.5kg batch, and may be considered representative of a clinical trial manufacturing facility. For many products, 500L could be the commercial scale ICB. The plant is capable of producing up to 30kg batches or about 0.5 tons per year.

Some products, however, need higher levels of production. The framework plant can expand by adding three 500L bioreactors in Stage Two, as well as more media and solution preparation and storage containers shown in Figure 4B. One remarkable feature of the common ICB framework is that the downstream remains unchanged, as discussed in the accompanying article (Coffman 2021). The “numbering up” of bioreactors does not change the regulatory filing. This Stage Two plant can produce up to 130kg batches and 2 tons per year (Figure 2).

The plant can expand further in Stage Three by adding more 500L bioreactors, or using the existing 500L as N-1 perfusion seed bioreactors to inoculate between one and six 2000L SUBs, though only four are shown in Figure 4C, and used in the examples.

More media and solution preparation and storage are also added. Remarkably, the downstream equipment remains the same [Coffman 2021]. Moving a product from 500L to 2000L bioreactors has a moderate regulatory impact and requires agency notification and comparability studies but should require no new clinical studies. This plant can produce 500kg batches in 20 days and up to 8 tons per year. The capacity is limited by the solution and media preparation and storage, and, to a lesser extent, the maximum flow rate of the capture step [see Coffman 2021]. The plant can operate either the 500L or the 2000L SUB as the N-stage production bioreactor to allow legacy production. The plant also supports legacy fed-batch production as well, with some restrictions on the harvest step.

As shown in Figure 4C, the 500kg batch requires only 80,000L of media concentrates, which can easily fit into 16 x 5,000L bags. This bag size is suggested because it is the largest single-use container available today. The 500kg lot would require about 24 x 5000L bags for the downstream, which is a manageable number.  The downstream solution bags require refilling once during the run. Media and solution preparation and storage is therefore not a major consideration for the framework process in new facilities.  It is a major consideration for retrofitting legacy facilities.

A bioreactor productivity of 2.5g/L/day, which is more routinely achievable today, could use six bioreactors running in parallel and produce a 300kg batch, or 5 tons per year. The total media and solution preparation remains the same, about 250,000L, in a 20-day period. As opposed to the 500kg batch, where four bioreactors were used, this operation requires more media preparation and less downstream solution preparation, thus balancing out the total volume prepared during the batch. The perfusion volume from the combined effluent from the six bioreactors is 12.5L/min.

## Increasing Buffer Preparation

Since solution preparation is limiting, there are good technologies that can help. If ILC is used on the chromatography and DF steps, the floor plan is much simpler. The solution PMI for downstream goes from 400 to 160 (Table 3). The 8-ton plant with ILC could look like Figure 2D. The media is contained in 16x5000L SU bags; the concentrates for ILC are in 5x5000L SU bags, which require a refill about twice during the run. The solution management is much simpler with ILC than with ILD. ILC can increase the capacity of the plant beyond 500kg/batch and 8 tons per year.

Water production is also a consideration. The total amount of water used to make up and dilute the media and solution concentrates over the course of 20 days is 1 ML, or about 50,000 L per day. This volume is not large for large scale facilities. The largest rate of water production would be required for the initial media and solution preparation, which can easily be spread out over the duration of the run. Thus, the water consumption rate for media preparation is not a major factor in this 4x2000L design for new facilities. It may be for retrofitting facilities.

The capital required for such a plant is relatively small. Jagshies correlation  [Jagshies, 2020], based on installed bioreactor volume, estimates the cost of the plant to be about $100M. Using a twenty-year depreciation schedule, the capital depreciation would contribute about $0.5/g to the COGM.  Media costs are the most likely to vary from company to company. As an example, a media that costs $20/L (Xu et al., 2017) would add $3/g to the overall COGM.  This cost seems manageable for most products. A full cost analysis of the common framework has not yet been published.

## Insensitivity to bioreactor productivity

The plant productivity is relatively insensitive to bioreactor productivity when the bioreactor productivity is above 2.3g/L/day, as shown in Figure 6. At this point, the plant becomes limited by solution preparation capacity, as well as the maximum flow rate a of the capture step. With productivities greater than 2.3g/L/day, the total solution preparation volume cannot make enough media and solutions to supply the downstream, so less bioreactor volume is used. The space for the media preparation allows more solution preparation. The 4.8g/L/day productivity used in the example allows for only four of the six bioreactors to be run. While leaving bioreactors empty might be counter intuitive, the cost of not running the bioreactor is much less than the cost of not fully using the downstream and solution preparation capacity of the plant. The flexibility of running more or fewer bioreactors allows the plant to be much less sensitive to the vagaries of cell culture development. As shown in Figure 6, the plant will produce between 5.5 tons per year and 8.4 tons per year over a cell productivity range of 2.3g/L/day to 8.3g/L/day , nearly a factor of four! In this range of productivities, the plant is near 100% utilization of all the equipment, except the bioreactors, which cost less than $1M each. The idling of $1-2M of capital is a small price to pay to enable the full utilization of the other $99M of capital.

Most companies can achieve a bioreactor productivity of 2.3g/L/day. This “entry-level” productivity allows huge increases in plant capacity for any network.

## Sustainability Framework

The integrated continuous process can take advantage of intensification, single use and multi column chromatography to show the lowest process mass intensity for mAb production.   The water intensity of biologics production in stainless steel has PMI ranging between 7000-10000 (Cataldo et al., 2020). Replacing stainless steel with single use reduces water (used for cleaning) by 70% and lowers the PMI by 60%.  Further PMI reductions, to values around 2000 (Table 3) are achieved by the continuous framework.

The electrical energy consumption from HVAC is a dominating factor in the life cycle assessment of bioprocesses, consuming about 70kW hr/kg of product (Cataldo et al., 2020)(Pietrzykowski et al., 2013). The energy consumption is reduced by the continuous framework by shrinking the footprint by 50%. The use of fully closed single use integrated processing can support the ball room classification grade reduction from C to D and provide an electrical energy reduction by 15-20%. Further improvements to the PMI are expected with the reduction in plastics and packaging waste by improved reuse and recycling that will support a circular economy in the coming years (Barbaroux et al., 2020).  A full environmental analysis of the common ICB framework is required.

# Conclusions

An 8-ton plant built on ICB and with single-use bioreactors is unprecedented. It rivals the largest fed-batch suites. While all the equipment necessary can be built from existing components, many improvements can be developed. The plant is backward compatible with fed-batch operations, although a process with 10g/L titer would only produce about 1 ton per year.

The volume of bioreactor(s) that the common framework can support is limited by media and solution storage, and the maximum flow rate of the capture step. In these examples, the maximum flow rate was set to 13L/min arbitrarily to yield a 500kg batch. Downstream skids with maximum flow rates of 50 L/min are common in large stainless-steel plants. If such equipment were used, the downstream could support a total bioreactor volume of 48,000L at 1.5vvm, and yield a lot of over 2.5 tons in twenty days. While no product today has such requirements, therapies for wide-spread diseases like Alzheimer’s and COVID-19 may require massive amount of material.

The impact of the manufacture of SARS-CoV-2 antibodies has caused a shortage of capacity for many years in the future. We may have to greatly and quickly expand the manufacturing capacity in order to supply the world with lifesaving antibodies to SARS-CoV-2. Or we may not, if the treatment does not work or vaccines provide adequate protection. The common framework for ICB allows a staged investment in new facilities and can grow with market demands, thus mitigating this risk against an uncertain future. The framework ICB allows expansion of the global biomanufacturing essentially at will, for very little capital, in nearly any place in the world.

# References

Alldread, R.M., Birch, J.R., Metcalfe, H.K., Farid, S., Racher, A.J., Young, R.J., and Khan, M. (2014). Large scale suspension culture of mammalian cells. In Industrial Scale Suspension Culture of Living Cells, H.-P. Meyer, and D.R. Schmidhalter, eds., pp. 410-462.

Angarita, M., Müller-Späth, T., Baur, D., Lievrouw, R., Lissens, G., and Morbidelli, M. (2015). Twin-column CaptureSMB: a novel cyclic process for protein A affinity chromatography. Journal of Chromatography A 1389, 85-95.

Arnold, L., Lee, K., Rucker-Pezzini, J., and Lee, J.H. (2019). Implementation of Fully Integrated Continuous Antibody Processing: Effects on Productivity and COGm. Biotechnology Journal 14.

Barbaroux, M., Horowski, B., Mokuolu, S., Petrich, M.A., and Whitfortd, W. (2020). Cycle Assessment and Sustainability for Single-Use Technologies in the Biopharmaceutical Industry. Bioprocess International.

Bettinardi, I.W., Castan, A., Medronho, R.A., and Castilho, L.R. (2020). Hydrocyclones as cell retention device for CHO perfusion processes in single-use bioreactors. Biotechnol Bioeng 117, 1915-1928.

Bolton, G.R., and Apostolidis, A.J. (2017). Mechanistic modeling of the loss of protein sieving due to internal and external fouling of microfilters. Biotechnology Progress 33, 1323-1333.

Brower, M., (Merck MSD), Charlebois, T.P., Connell-Crowley, L.J.T., Erikson, J.G., NIIMBL), Hart, R.A., Jagshies, G.C., f.k.a GE Healthcare), Paolella, D.G., Pollard, D.S., Schaefer, E.J.J., Schultz, J.N., et al. (2020).

Brower, M., Hou, Y., and Pollard, D. (2015). Monoclonal antibody continuous processing enabled by single use. In Continuous Processing in Pharmaceutical Manufacturing, G. Subramanian, ed., pp. 255-296.

Brown, A., Bill, J., Tully, T., Radhamohan, A., and Dowd, C. (2010). Overloading ion-exchange membranes as a purification step for monoclonal antibodies. Biotechnol Appl Biochem 56, 59-70.

Budzinski, K., Blewis, M., Dahlin, P., D'Aquila, D., Esparza, J., Gavin, J., Ho, S.V., Hutchens, C., Kahn, D., Koenig, S.G., et al. (2019). Introduction of a process mass intensity metric for biologics. New Biotechnology 49, 37-42.

Carredano, E.N., Nordberg, R., Westin, S., Busson, K., Karlsson, T.M., Blank, T.S., Sandegren, H., and Jagschies, G. (2018). Chapter 27 - Simplification of Buffer Formulation and Improvement of Buffer Control with In-Line Conditioning (IC). In Biopharmaceutical Processing, G. Jagschies, E. Lindskog, K. Łącki, and P. Galliher, eds. (Elsevier), pp. 513-525.

Cataldo, A.L., Burgstaller, D., Hribar, G., Jungbauer, A., and Satzer, P. (2020). Economics and ecology: Modelling of continuous primary recovery and capture scenarios for recombinant antibody production. J Biotechnol 308, 87-95.

Clincke, M.F., Molleryd, C., Zhang, Y., Lindskog, E., Walsh, K., and Chotteau, V. (2013). Very high density of CHO cells in perfusion by ATF or TFF in WAVE bioreactor. Part I. Effect of the cell density on the process. Biotechnol Prog 29, 754-767.

Dennis M. DiSorbo, D.W.J. (1995). Media Concentrate Technology, USPO, ed.

Gillespie, C., Holstein, M., Mullin, L., Cotoni, K., Tuccelli, R., Caulmare, J., and Greenhalgh, P. (2019). Continuous In-Line Virus Inactivation for Next Generation Bioprocessing. Biotechnol J 14, e1700718.

Hummel, J., Pagkaliwangan, M., Gjoka, X., Davidovits, T., Stock, R., Ransohoff, T., Gantier, R., and Schofield, M.J.B.j. (2019). Modeling the downstream processing of monoclonal antibodies reveals cost advantages for continuous methods for a broad range of manufacturing scales.  14, 1700665.

Ichihara, T., Ito, T., and Gillespie, C. (2019). Polishing approach with fully connected flow-through purification for therapeutic monoclonal antibody. Eng Life Sci 19, 31-36.

Ichihara, T., Ito, T., Kurisu, Y., Galipeau, K., and Gillespie, C. (2018). Integrated flow-through purification for therapeutic monoclonal antibodies processing. MAbs 10, 325-334.

Kelley, B. (2007). Very Large Scale Monoclonal Antibody Purification: The Case for Conventional Unit Operations. Biotechnol Prog 23, 995-1008.

Kelley, B. (2009). Industrialization of mAb production technology: the bioprocessing industry at a crossroads. MAbs 1, 443-452.

Kelley, B. (2020). Developing therapeutic monoclonal antibodies at pandemic pace. Nat Biotechnol 38, 540-545.

Kelly, W., Scully, J., Zhang, D., Feng, G., Lavengood, M., Condon, J., Knighton, J., and Bhatia, R. (2014). Understanding and modeling alternating tangential flow filtration for perfusion cell culture. J Biotechnology progress 30, 1291-1300.

Klutz, S., Holtmann, L., Lobedann, M., and Schembecker, G.J.C.E.S. (2016a). Cost evaluation of antibody production processes in different operation modes.  141, 63-74.

Klutz, S., Lobedann, M., Bramsiepe, C., and Schembecker, G. (2016b). Continuous viral inactivation at low pH value in antibody manufacturing. Chemical Engineering and Processing: Process Intensification 102, 88-101.

Kwon, T., Prentice, H., Oliveira, J., Madziva, N., Warkiani, M.E., Hamel, J.P., and Han, J. (2017). Microfluidic Cell Retention Device for Perfusion of Mammalian Suspension Culture. Sci Rep 7, 6703.

Levine, H.L., Stock, R., Lilja, J., Gaasvik, A., Hummel, H., Ransohoff, T.C., and Jones, S.D.J.B.I. (2013). Single-use technology and modular construction.  11, 40-45.

Li, F., Vijayasankaran, N., Shen, A.Y., Kiss, R., and Amanullah, A. (2010). Cell culture processes for monoclonal antibody production. MAbs 2, 466-479.

Liu, H.F., McCooey, B., Duarte, T., Myers, D.E., Hudson, T., Amanullah, A., van Reis, R., and Kelley, B.D. (2011). Exploration of overloaded cation exchange chromatography for monoclonal antibody purification. J Chromatogr A 1218, 6943-6952.

Martins, D.L., Sencar, J., Hammerschmidt, N., Flicker, A., Kindermann, J., Kreil, T.R., and Jungbauer, A. (2020). Truly continuous low pH viral inactivation for biopharmaceutical process integration. Biotechnol Bioeng 117, 1406-1417.

Parker, S.A., Amarikwa, L., Vehar, K., Orozco, R., Godfrey, S., Coffman, J., Shamlou, P., and Bardliving, C.L. (2018). Design of a novel continuous flow reactor for low pH viral inactivation. Biotechnol Bioeng 115, 606-616.

Pietrzykowski, M., Flanagan, W., Pizzi, V., Brown, A., Sinclair, A., and Monge, M. (2013). An environmental life cycle assessment comparison of single-use and conventional process technology for the production of monoclonal antibodies. Journal of Cleaner Production 41, 150-162.

Pinto, N.D.S., and Brower, M. (2020). Wide-surface pore microfiltration membrane drastically improves sieving decay in TFF-based perfusion cell culture and streamline chromatography integration for continuous bioprocessing. Biotechnol Bioeng.

Pinto, N.D.S., Napoli, W.N., and Brower, M. (2020). Impact of micro and macroporous TFF membranes on product sieving and chromatography loading for perfusion cell culture. Biotechnology and Bioengineering 117, 117-124.

Pollock, J., Bolton, G., Coffman, J., Ho, S.V., Bracewell, D.G., and Farid, S.S.J.J.o.C.A. (2013a). Optimising the design and operation of semi-continuous affinity chromatography for clinical and commercial manufacture.  1284, 17-27.

Pollock, J., Ho, S.V., and Farid, S.S. (2013b). Fed‐batch and perfusion culture processes: economic, environmental, and operational feasibility under uncertainty. Biotechnology and Bioengineering 110, 206-219.

Pollock, J., Ho, S.V., Farid, S.S.J.B., and bioengineering (2013c). Fed‐batch and perfusion culture processes: economic, environmental, and operational feasibility under uncertainty.  110, 206-219.

Radoniqi, F., Zhang, H., Bardliving, C.L., Shamlou, P., and Coffman, J. (2018). Computational fluid dynamic modeling of alternating tangential flow filtration for perfusion cell culture. Biotechnol Bioeng 115, 2751-2759.

Reck, J.M., Pabst, T.M., Hunter, A.K., and Carta, G. (2017). Separation of antibody monomer-dimer mixtures by frontal analysis. J Chromatogr A 1500, 96-104.

van Reis, R., and Zydney, A. (2007). Bioprocess membrane technology. Journal of Membrane Science 297, 16-50.

Vogg, S., Pfeifer, F., Ulmer, N., and Morbidelli, M. (2020). Process intensification by frontal chromatography: Performance comparison of resin and membrane adsorber for monovalent antibody aggregate removal. Biotechnol Bioeng 117, 662-672.

Walther, J., Godawat, R., Hwang, C., Abe, Y., Sinclair, A., and Konstantinov, K. (2015). The business impact of an integrated continuous biomanufacturing platform for recombinant protein production. Journal of Biotechnology 213, 3-12.

Wang, S., Godfrey, S., Ravikrishnan, J., Lin, H., Vogel, J., and Coffman, J. (2017). Shear contributions to cell culture performance and product recovery in ATF and TFF perfusion systems. J Biotechnol 246, 52-60.

Wang, S.B., Godfrey, S., Radoniqi, F., Lin, H., and Coffman, J. (2019). Larger Pore Size Hollow Fiber Membranes as a Solution to the Product Retention Issue in Filtration-Based Perfusion Bioreactors. Biotechnol J 14, e1800137.

Warikoo, V., Godawat, R., Brower, K., Jain, S., Cummings, D., Simons, E., Johnson, T., Walther, J., Yu, M., Wright, B., et al. (2012). Integrated continuous production of recombinant therapeutic proteins. Biotechnology and Bioengineering 109, 3018-3029.

Wolf, M.K.F., Bielser, J.M., and Morbidelli, M. (2020). Perfuson Cell Culture Process for Biopharmaceuticals (Cambridge: Cambridge University Press).

Xu, S., Gavin, J., Jiang, R., and Chen, H. (2017). Bioreactor productivity and media cost comparison for different intensified cell culture processes. Biotechnol Prog 33, 867-878.

Zydney, A.L. (2016). Continuous downstream processing for high value biological products: A Review. Biotechnology and Bioengineering 113, 465-475.