# The Design Basis for the Common Framework for Integrated and Continuous Bioprocessing

© 2020 AstraZeneca

Jon Coffman, Nicholas Guros, Rick Lu, Ujwal Patil, Steve Rose, Mark Brower*

*Merck and Company, Kennilworth, NJ

# Abstract

An 8-ton per year manufacturing facility is described based on the framework for integrated and continuous bioprocessing (ICB) common to all known biopharmaceutical implementations. While the output of this plant rivals some of the largest fed-batch plants in the world, the equipment inside the plant is relatively small: the plant consists of four 2,000L single use bioreactors and has a maximum flow rate of 13 L/min.  The equipment and facility for the ICB framework is described in sufficient detail to allow biopharmaceutical companies, vendors, contract manufacturers to build or buy their own systems. The design will allow the creation of a global ICB ecosystem that will transform biopharmaceutical manufacturing.

The design is fully backwards compatible with legacy fed-batch processes.  A clinical production scale is described that can produce smaller batch sizes with the same equipment as that used at the commercial scale. The design described allows the production of as little as 100g/day to nearly 40kg per day.

# Introduction

Many biopharmaceutical companies are investigating the next generation of biologics manufacturing by developing integrated and continuous bioprocessing (ICB).  A seen in the companion article, a common framework for ICB exists that would enable most, if not all, ICB processes currently being implemented. It can be implemented in a staged approach in new facilities.

The plant and equipment designs described here can produce quantities of recombinant proteins that would satisfy any market need.  Very few products require over 1 ton of material per year (Kelley, 2009); the framework design is capable of 8 tons per year.

The examples include a 500L bioreactor with a low productivity cell line and 2000L bioreactors with a high productivity cell line. The 500L bioreactor makes about 0.5kg of material in the bioreactor per day, which results after 20 days in a 6kg batch. The 2000L bioreactor makes about 10kg of material per day, which results after 20 days in a 130kg batch. A four 2000L process is described that results in as much as a 500kg batch over 20 days, or about 8 tons per year in 17 batches.

The world may have an urgent need for quickly expandable manufacturing space. The common framework enables single-use equipment to be as productive as larger stainless-steel plants. Due to the COVID-19 pandemic, many companies are buying up existing capacity in large stainless-steel facilities for their antibody treatments. Expanding these facilities will take time. In addition, these facilities cost $400-800M (Jagschies, 2020). Finally, due to political nationalism, many countries and regions want to control the manufacture of these COVID-19 treatments.  By enabling cost-effective use of single-use technologies, the common framework allows the fast, and relatively inexpensive, building of large-capacity biomanufacturing.

# Materials and Methods

The design of the upstream and downstream was based on common and well-known engineering principles. The assumptions are shown in Table 1. The justification for the assumptions, and a more detailed materials and methods, is in the supplementary material. In addition, a spreadsheet is enclosed in the supplementary material that can enable other manufacturing scenarios. A crowd-sourcing spreadsheet is located online, which will allow further development of new technologies (Coffman, 2020).

# Results and Discussion

The high-level consequences of the common framework ICB are discussed in the accompanying article (Coffman, 2021). Here we discuss the details of the design to demonstrate the flexibility and productivity of the framework ICB. The examples used are shown in Figure 1 and Tables 1 and 2. They are described in more detail below.

## Bioreactor Design

The common framework bioreactor is designed to support high cell densities. The largest design challenges deal with supplying cells with O2 and stripping cells of CO2. The mass of nutrients required by high cell densities requires the development of media concentrates, as well as the possibility of several feed solutions. The high density of cells also requires cooling to maintain the temperature. While these challenges impact the bioreactor design in several ways, they do not preclude the bioreactor for standard fed-batch operation.

Dynamic, or non-steady-state perfusion, where the cell density is not directly controlled by a bleed, can reach cell densities of 100-150Mcells/mL (Wolf et al., 2020). These cell densities would require oxygen transfer rates between 15 and 50 mM/hour (Jorjani and Ozturk, 1999). The gas/liquid transfer coefficient kLa  between 15 and 70 hour-1 would be required (Moutafchieva et al., 2013).  The cells consume oxygen so fast that they become hypoxic in less than 10 seconds without sparging (SI). Further, there are a few lab or pilot scale examples of perfusion cell culture at 240Mcells/mL (Clincke et al., 2013a; Clincke et al., 2013b; Zamani et al., 2018). If the industry is to expand to this cell density, a kLa of over 100 hr-1, or lower cell-specific oxygen uptake rates, would be required.

The gassing strategy for a perfusion culture is a three-fold balancing act between control of dissolved oxygen (DO), pCO2 accumulation (i.e. pH and subsequent base utilization), and foam formation. Achieving desired control in any one aspect can result in an undesirable result in another aspect.

The supply of oxygen to the cells can easily be the limiting factor in achieving high cell densities (Ozturk, 1996, Zhu, Marie et al. 2017).  The design of the mass-flow controller and sparger require consideration. Generally, mass-flow controllers delivering air and O2 for perfusion processes requires significantly higher flow capacity than fed-batch processes, ranging from 0.05vvm to 0.2vvm. With respect to sparger design, micro-spargers have been demonstrated to improve the oxygen transport capacity needed to achieve a desired DO at perfusion-scale cell densities (Diekmann et al., 2011; Dreher et al., 2014). However, microspargers raise several concerns, including their potential to cause cell damage due to shear when the microscopic bubbles burst (Wolf et al., 2020). Microspargers have also been observed to yield higher pCO2 accumulation than traditional macro, i.e. ring or drilled-hole, spargers (Dreher et al., 2014), which lowers culture pH, and subsequently more base utilization which can be increase cell stress and the rate of cellular apoptosis. Additionally, microsparging creates a thicker layer of foam than traditional spargers. This thick foam requires enhanced control methods, and is a concern with respect to fouling bioreactor exhaust.

A dual-sparger design that is composed of a micro and a macro sparger permits the most flexibility in configuring the gas flows to balance the requirements for dissolved oxygen, dissolved carbon dioxide, and foam management in a perfusion bioreactor. In this design, the microsparger component generally provides the required dissolved oxygen to a culture via relatively smaller bubbles and the macrosparger component generally provides a means to manage pCO2 accumulation via stripping carbon dioxide from the system with relatively larger air bubbles. The dual-sparger design can also provide an additional benefit of reducing the amount of foam generated via destruction interactions between the two bubble sizes (Karakashev, Stoyan et al. 2012).

Even with a dual sparger design, foam and aerosol management is more difficult in perfusion cultures due to relatively higher agitation and gas flow rates than fed-batch cultures. The first line of defense is to manage foam generation in the bioreactor itself through means such as mechanical disturbance, adjustment of overlay humidity, and automated addition of antifoaming agents as a function of the rate of foam generation and foam thickness (Proulx et al., 2019). Redundant filters and/or vent traps are essential components of the exhaust manifold to manage the collection of foam and aerosols, as is the ability to seamlessly switch between such unit operations while maintaining a sterile boundary. Advanced filtration systems (Le Merdy et al., 2017; Pegel et al., 2011) demonstrate an enhanced ability to manage foam and aerosol generation, and should be considered for implementation in any bioreactor exhaust manifold.

The last consideration for the perfusion bioreactor gassing strategy is the control strategy. Essential to maintaining the balance between DO, pCO2 accumulation, and foam generation is establishing bioreactor-specific relationships for the parameters that govern this balance, including the gas transfer (i.e. kLa) for a given gas species and sparger design. Therefore, it is essential for perfusion bioreactor control architecture to permit the use of reasonably complex algorithms (Abbate et al., 2020) to manage these three control elements, as well as the ability to incorporate so-called offline, at-line, and online measurements such as cell density, temperature, lactic acid concentration, glucose concentration, and pH.

Media delivery is also different for high-cell density perfusion compared to fed-batch. The media is concentrated, and can be in multiple streams rather than one stream. In the scenarios shown in Table 1, the media concentrates are made up of a 5x, 12x, and 20x concentrates, which, taken as a whole, are effectively a 3x concentrate. Multiple media feed pumps sized from 0.02 vvd to 0.3vvd would be needed. For a 2000L bioreactor, this is a surprisingly small 20-400mL/min. A glucose concentrate and antifoam will also need to be added with even smaller flow rates. The pump rates should be capable of being set by complicated calculations that may rely on PAT or on mechanistic models, similar to the ones that might be required for gassing.  Overall bioreactor level control is required to control the feeds or the permeate rate.

The media concentrates may be added through a single line with a static mixer. Typically, a line of water enters the static mixer, and a series of T-connections allow the mixing of each concentrate in a serial fashion.

The use of media concentrates greatly reduces the size of the media preparation and media storage area. In the 4x2000L scenario shown in Tables 1 and 2, the media can be stored in sixteen 5,000L single use containers. The 20x media would require an additional 2,000L container.

These bioreactors may become exothermic and require cooling due to the exceptional quantity of cell mass, a novel concern for perfusion as compared to batch mammalian cell culture. Cells generate about 28pW/cell (Kemp and Guan, 1997) or between 1.4 to 4.2 W/L for cell densities between 50M-150 Mcells/mL. A 2000L vessel could generate between 3000W to 8000W, which is easily handled by cooling jackets. Cooling a 12kL bioreactor may, however, become a limitation as the cells could generate up to 50kW. A heat exchanger in line with the tangential flow filtration system may become necessary.

A design that might be suitable for a clinical manufacturing facility is shown in Column 1 in Tables 1 and 2. The 500L bioreactor operates at 50Mc/mL with a 20 pg/c/day cell line. This scenario represents one that can be achieved by converting a fed-batch process over to perfusion cell-culture without much difficulty. It also represents one that might occur early in development, when process optimization and intensification has not occurred. The bioreactor makes 500g/day.

A commercial bioreactor set up is shown in Table 1 and 2, and column 3, operating at an average cell density of 120Mc/mL with a 40pcd cell line. In this scenario, four 2,000L bioreactors are operated to feed a single downstream. The bioreactors together produce 38kg per day.  Over the course of the 20 day run, over 500kg of material is pooled into a single lot.

## Cell Retention Design

The TFF design is shown in Table 1 and 2 for a 500L and 2000L bioreactor.  In order to mitigate against Starling flow (Radoniqi et al., 2018), the membrane area was scaled up by adding filter modules in series, rather than in parallel.  The 2000L bioreactor requires four hollow fiber modules in series (see Figure 2A). A parallel scale-up strategy would require the retentate flow rate to between 500 and 1000L/min, or 0.25-0.5vvm, Figure 2B. These flow rates would require very large pumps, and the cells would have 5-10 times more pump passes. Damage to cells through pumps is an important factor to mitigate (Wang et al., 2017).

Switching the filter for the 2000L bioreactor design is best accomplished by the use of Y-connectors and valves between the two sets of filter modules. Ideally, the connectors would have zero dead volume, so cells do not get stuck in the dead leg. Manual switching between the two sets of filter modules is likely acceptable for most operations, even in commercial space.

The design calls for one ATF10 systems for the 500L bioreactor and three per 2000L bioreactor. While this takes up a considerable amount of floor space around the bioreactor, it is not a major inconvenience.

The area required for both ATF and TFF on the 2000L bioreactor is 30m2. Modules are commercially available. The ATF and TFF systems require three or four small permeate pumps with a maximum flow rate of less than 500 mL/min. The total flow rate onto the capture step is small, about 8L/min for the 4x2000L design.  It should be noted that the residence time of the cells in the filters is near or exceeds the time to hypoxia.

Single-use bioreactors over 500L do well to have  1.5-inch to 2-inch ports to allow unrestricted flow for each pump system. Connecting the SUB to the TFF requires a 1.5-inch sanitary connection, which is not known to be commercially available. Instead, two 1-inch ports are often used for each connection. The 2000L bioreactor requires four 1.5 to 2-inch ports to support the ATF and two 1.5 to 2-inch ports to support the TFF operation. The common framework bioreactor supports both ATF and TFF operation.

## Dual Column Chromatography Design

Dual chromatography is the simplest and perhaps easiest form of ICB that can be implemented (Angarita et al., 2015). The high-level skid design shown in Figure 3 allows flexibility for ICB and batch processing. The system uses two pump sizes. A smaller pump is used for the solution concentrates. Larger pumps are used for the diluent and the load.  This design has a large processing range. The overall accurate flow rate range is 0.3 to 12.5 L/min. High productivity processes use solution concentrates and dilute with water. The largest processing possible on this system is two 25L columns operated in series, allowing for 57.6kg/day. The smallest mass that can be processed uses only the smaller pump with no dilution at the lowest accurate flow rate, allowing the use of a 0.6L column. This set up processes as little as 36g per day.  Two columns operated in parallel can process as much as 48kg/day, due to the decreased effective capacity of the column. For plants that desire this degree of flexibility, the product flow path on such a skid would be single use, and allow a high-flow and a low-flow tubing kit to avoid product dilution at the low flow rate range.  Most plants would not need this degree of flexibility and the full range of productivity will depend upon the tubing diameter and the extra-column volume.

The downstream process is tolerant to changes in productivity in the upstream in two ways. Firstly, the columns can be underloaded. The load range the Protein A is assumed to be 10g/L-72g/L. The second is that the columns can be cycled as few as once per day, to as many as sixteen times per day. The lower limit is set, in part, by the bioburden control strategy, in that we prefer to load a column less than 12 hours before sanitizing it. The total in-batch range of productivity is therefore 230 fold. Between batches, the columns can be changed in size from 0.6L to 25L. Thus, the overall batch-to-batch productivity range for the downstream is 9600-fold range.

The average number of cycles per day is set to avoid changing columns during the 20-day run for commercial processes. Each column is cycled 10 times per day, for a total cycling of 200 cycles per lot. While column lifetime may vary from product to product, targeting 200 cycles is not overly burdensome for commercial processes. Poorer column lifetime will require larger columns and fewer cycles per day.

The system also has UV absorbance detection with two path lengths to allow both high and low concentration measurement. Ideally, the UV detector would be linear up to concentrations of 150 g/L, or about 200 AU/cm. A detector with 0.1mm and 1.0mm path lengths would be sufficient. These detection limits are necessary to enable accurate yield calculations on every cycle. Optionally, the detector would allow a scan of absorbance from 230 nm to 320 nm. Spectral analysis of column peaks can be used to estimate purity (Brestrich et al., 2014; Brestrich et al., 2016; Brestrich et al., 2015). Additional data connections should be available for expanding PAT methods that are not described here.

The dual column chromatography skid may not benefit from being single use, depending upon the application. Small lot sizes will have an increased burden of on the COGM of the single use assemblies. This burden is less important in clinical manufacturing, where flexibility and change-over time might be more important. This burden is also less important for very large batches, such as those derived from the 4x2000L example. Plants producing more material than the 4x2000L example would be limited by the available SU column sizes, large SU valves, and the number of SU containers for media and solution preparation and storage. SU pumps are not limiting:  A system based on commonly used single use pumps (Quatroflo 4400SU, 83L/min max, and Quatroflo 1200SU, 20L/min max) could conceivably be used to process nearly 400kg/day.

The dual column system is backward compatible with batch processing. The downstream can process 48kg/day, or from a fed-batch with 24g/L titer, from each of the four bioreactors if they are harvested on different days. This titer is well in excess of any standard fed batch process.

## Virus Inactivation

The framework virus inactivation step supports both batch and continuous virus inactivation. The skid performs on-line titration of the peak as it elutes from the Protein A step. The acidified peak then goes into a tank for the batch operation or into a plug flow reactor (PFR) for continuous inactivation. At the end of the incubation time, the product stream is neutralized before passing to the next step.

Those processes that use a surge tank prior to the virus inactivation step will have a homogeneous product stream entering and exiting the PFR. This homogeneity allows for either a volume-based titration, or one based on a closed-loop pH control. Those steps that titrate the peak in-line without a surge tank will require some feed-forward control based on UV. The UV signal from the Protein A skid will require integration with the virus inactivation skid, or the VI skid will require a UV sensor on the inlet.

## Depth Filtration

High-density fed-batch processes precipitate HCP and other impurities after the low pH virus inactivation process. The particulates are removed typically by a depth filter.

Perfusion based processes do not precipitate with the same frequency or degree. The framework process requires the capability of adding a depth filter. The depth filter has challenges for continuous processing. There are not many depth filters that arrive sterile or have a low bioburden specification. This means that depth filtration is an opportunity for bioburden to enter the process. Some depth filters can be sanitized with various solutions, hydroxide being the most typical.

Since the depth filter capacity is so low (100 L/m2), it will require frequent changing. The framework presumes daily changing.  The filter area is proportional to the Protein A column volume, and varies from 0.5 m2 for the clinical process to 13m2 for the 500kg process. While 13m2 is seen commonly in manufacturing plants, it is large and is best flushed with its own pump system.

## Virus retentive filtration (VRF)

Virus filtration provides a robust and scalable solution for viral clearance in biologic manufacturing. The choice of a suitable virus filter is governed by a combination of feed characteristics and process parameters. Therefore, the lack of in-depth understanding has, for many, restricted the VRF process to a batch operation. Table 1 and 2 depict various scenarios for a VRF process following the second polishing step. As shown in Table 2, the low productivity scenario would necessitate a membrane area of 0.5 m2. Each VRF sub-batch is expected to be operated at 24 h intervals. Here, each sub-batch will take roughly three hours, yielding the final product pool of 277 Litres at 24 g/L (Table 2). The concomitant adjustments in membrane area and other operating parameters towards changes in capacity and productivity are mentioned in the supplementary information (Table 2 & SI Table).

## Ultrafiltration and Diafiltration (UF/DF)

The common framework process enables both batch and single-pass (SP) ultrafiltration and diafiltration (UFDF). The framework uses a flexible single-use skid with the same pumps and different flow path to accommodate both operations.

The batch UF/DF step is shown in Figure 5 and involves batch tangential flow filtration (TFF) and a recirculation skid to attain the desired product concentration. The framework process performs a batch UFDF frequently to avoid the need for a large system.  The entire UF/DF process will be split into as many as 20 sub-batches that are expected to be in-sync with the VRF process. A concentration step is operated continuously as the run progresses. Periodically, the concentrated pool is moved to the DF retentate tank where the diafiltration is performed. The material is moved to the final in-process retentate pool where the final UF concentration occurs continuously as the run progresses. Table 1 and 2, depicts the process parameters and throughput for a typical TFF module.  The size of the filters and pumps used is similar across all scales by controlling the frequency of the batch operation. The filter used for the clinical 6 kg DF is 0.2-1 m2 and for the commercial 500kg batch is 0.4-4m2. Each are sub batched one time and twenty times, respectively.

The volume of DF buffer required for a batch with high productivity can be prohibitively high (Table 2 and SI Table). For the 500kg batch, the DF buffer volume is 66,000L, which is 25% of the total downstream solution volume. This issue can be partially mitigated by the use of buffer concentrates or in-line conditioning. Accurate pumps are needed for this final step.

Recently, alternative approaches to overcome shortfalls of the typical UF/DF operation have been reported in the literature. The advent of single-pass tangential flow filtration (SPTFF) membrane filtration modules has made it possible to attain high protein concentration factors in a ‘single-pass’ while eliminating the need for repeated product recirculation (Jungbauer, 2013). The use of multistage SPTFF cascade for continuous diafiltration of proteins using cocurrent and countercurrent mode have been demonstrated (Arnold et al., 2019; Huter and Strube, 2019; Jabra et al., 2019; Nambiar et al., 2018; Rucker-Pezzini et al., 2018). Table 2 describes the proposed implementation of the SPTFF modules for an integrated and continuous UF and countercurrent-DF (CC-DF) process. Here, the feed flow to the UF process is adjusted in a way to spread out each VRF sub-batch pool over 24 h. This allows the UF process to be operated uninterrupted over the entire batch. The retentate stream is then directly fed to the three-stage CC-DF cascade eliminating the need for a surge tank. The DF buffer's countercurrent flow results in at least a 40% reduction in buffer usage than batch DF operation, primarily because the CC-DF step operates at a higher product concentration.

A few pumps are mismatched between the UFDF and SP options. Multihead peristaltic pumps can simplify the pump design considerably.  The pumps used on the module could use high-flow and low-flow tubing depending upon the application.

Recent reports also successfully demonstrated the use of hollow-fiber dialyzers for buffer exchange (Yehl et al., 2019). The framework skid would also enable the operation of this step, which has the potential of reducing the total DF buffer required by a factor of 8 from the batch-wise UFDF option.

# Conclusion

The common integrated and continuous bioprocessing framework enables the design of equipment flexible enough to process 6kg to 500kg lot sizes. The design also allows flexibility for some batch operations and is compatible with legacy processes.

This flexible, mostly single-use design has particular relevancy today during the pandemic. While most COVID-19 mAb projects are made in fed-batch right now, converting from fed-batch to ICB is possible. While converting to ICB will not help in the near term, it would allow manufacturing expansion within about two years. Converting to perfusion bioreactor would be a relatively large change from a regulatory perspective, and would require strong comparability. The alternative is a capacity crunch and investment in billions of dollars of stainless steel infrastructure in countries all over the world. Since SARS-CoV-2 is not going away, the antibody manufacturing may be required for many years to come. In addition, fast development of scalable mAb manufacturing will help in the next pandemic, which is closer than we would like to think.

The framework ICB allows companies to harmonize their platform around off-the-shelf equipment.  Also, it’s possible for contract manufactures to buy equipment that would be backward compatible with batch processes, as well as future compatible for ICB. This flexibility has not been realized to date. Many companies think of ICB processes as a step forward from which there is no turning back. This paper demonstrates that the common framework ICB can be designed, purchased, and installed with an eye toward the future, and yet without worry about the past.

Bakhshayeshi M, Zydney AL. 2008. Effect of solution pH on protein transmission and membrane capacity during virus filtration. Biotechnol. Bioeng. 100:108–117.

Bohonak DM, Zydney AL. 2005. Compaction and permeability effects with virus filtration membranes. J. Memb. Sci. 254:71–79.

Bolton G, Cabatingan M, Rubino M, Lute S, Brorson K, Bailey M. 2005. Normal-flow virus filtration: detection and assessment of the endpoint in bioprocessing. Biotechnol. Appl. Biochem. 42:133.

Ireland T, Bolton G, Noguchi M. 2005. Optimizing Virus Filter Performance  with Prefiltration. Bioprocess Int. 3:44–7.

Rathore AS, Kumar V, Arora A, Lute S, Brorson K, Shukla A. 2014. Mechanistic modeling of viral filtration. J. Memb. Sci. 458:96–103.

Strauss D, Goldstein J, Hongo-Hirasaki T, Yokoyama Y, Hirotomi N, Miyabayashi T, Vacante D. 2017. Characterizing the impact of pressure on virus filtration processes and establishing design spaces to ensure effective parvovirus removal. Biotechnol. Prog. 33:1294–1302.

Syedain ZH, Bohonak DM, Zydney AL. 2006. Protein Fouling of Virus Filtration Membranes: Effects of Membrane Orientation and Operating Conditions. Biotechnol. Prog. 22:1163–1169.

Tateishi J, Kitamoto T, Mohri S, Satoh S, Sato T, Shepherd A, Macnaughton MR. 2001. Scrapie Removal using Planova®Virus Removal Filters. Biologicals 29:17–25.

Wickramasinghe SR, Stump ED, Grzenia DL, Husson SM, Pellegrino J. 2010. Understanding virus filtration membrane performance. J. Memb. Sci. 365:160–169.

References for UF/DF

Arnold L, Lee K, Rucker‐Pezzini J, Lee JH. 2019. Implementation of Fully Integrated Continuous Antibody Processing: Effects on Productivity and COGm. Biotechnol. J. 14:1800061.

Huter MJ, Strube J. 2019. Model-Based Design and Process Optimization of Continuous Single Pass Tangential Flow Filtration Focusing on Continuous Bioprocessing. Processes 7:317.

Jabra MG, Yehl CJ, Zydney AL. 2019. Multistage continuous countercurrent diafiltration for formulation of monoclonal antibodies. Biotechnol. Prog. 35:6–11.

Jungbauer A. 2013. Continuous downstream processing of biopharmaceuticals. Trends Biotechnol. 31:479–492.

Nambiar AMK, Li Y, Zydney AL. 2018. Countercurrent staged diafiltration for formulation of high value proteins. Biotechnol. Bioeng. 115:139–144.

Rucker-Pezzini J, Arnold L, Hill-Byrne K, Sharp T, Avazhanskiy M, Forespring C. 2018. Single Pass Diafiltration Integrated into a Fully Continuous mAb Purification Process. Biotechnol. Bioeng.

Yehl CJ, Jabra MG, Zydney AL. 2019. Hollow fiber countercurrent dialysis for continuous buffer exchange of high-value biotherapeutics. Biotechnol. Prog. 35:1–5.

Abbate, T., Sbarciog, M., Dewasme, L., and Vande Wouwer, A. (2020). Experimental Validation of a Cascade Control Strategy for Continuously Perfused Animal Cell Cultures. Processes 8, 413.

Angarita, M., Müller-Späth, T., Baur, D., Lievrouw, R., Lissens, G., and Morbidelli, M. (2015). Twin-column CaptureSMB: a novel cyclic process for protein A affinity chromatography. Journal of Chromatography A 1389, 85-95.

Brestrich, N., Briskot, T., Osberghaus, A., and Hubbuch, J. (2014). A tool for selective inline quantification of co-eluting proteins in chromatography using spectral analysis and partial least squares regression. Biotechnol Bioeng 111, 1365-1373.

Brestrich, N., Hahn, T., and Hubbuch, J. (2016). Application of spectral deconvolution and inverse mechanistic modelling as a tool for root cause investigation in protein chromatography. Journal of Chromatography A 1437, 158-167.

Brestrich, N., Sanden, A., Kraft, A., McCann, K., Bertolini, J., and Hubbuch, J. (2015). Advances in inline quantification of co-eluting proteins in chromatography: Process-data-based model calibration and application towards real-life separation issues. Biotechnol Bioeng 112, 1406-1416.

Clincke, M.F., Molleryd, C., Samani, P.K., Lindskog, E., Faldt, E., Walsh, K., and Chotteau, V. (2013a). Very high density of Chinese hamster ovary cells in perfusion by alternating tangential flow or tangential flow filtration in WAVE Bioreactor-part II: Applications for antibody production and cryopreservation. Biotechnol Prog 29, 768-777.

Clincke, M.F., Molleryd, C., Zhang, Y., Lindskog, E., Walsh, K., and Chotteau, V. (2013b). Very high density of CHO cells in perfusion by ATF or TFF in WAVE bioreactor. Part I. Effect of the cell density on the process. Biotechnol Prog 29, 754-767.

Coffman, J. (2020). Common Framework Spreadsheet.

Diekmann, S., Dürr, C., Herrmann, A., Lindner, I., and Jozic, D. (2011). Single use bioreactors for the clinical production of monoclonal antibodies – a study to analyze the performance of a CHO cell line and the quality of the produced monoclonal antibody. Paper presented at: BMC Proceedings (BioMed Central).

Dreher, T., Husemann, U., Adams, T., de Wilde, D., and Greller, G. (2014). Design space definition for a stirred single-use bioreactor family from 50 to 2000 L scale. Engineering in Life Sciences 14, 304-310.

Jagschies, G. (2020). Hierarchy of High Impact Improvements in Bio Manufacturing. Paper presented at: Workshop on Innovations in Pharmaceutical Manufacturing (Washington, DC: National Academies of Science).

Jorjani, P., and Ozturk, S.S. (1999). Effects of Cell Density and Temperature on Oxygen Consumption Rate for Different Mammalian Cell Lines. Biotechnology and Bioengineering 64, 349-356.

Kelley, B. (2009). Industrialization of mAb production technology: the bioprocessing industry at a crossroads. MAbs 1, 443-452.

Kemp, R.B., and Guan, Y. (1997). Heat flux and the calorimetric-respirometric ratio as measures of catabolic flux in mammalian cells. Thermochimica acta 300, 199-211.

Le Merdy, S., Cherradi, Y., and Kalashnikov, P. (2017). Exploration of synthetic depth filtration applied to mammalian cell harvest (Merck Millipore).

Moutafchieva, D., Popova, D., Dimitrova, M., and Tchaoushev, S. (2013). EXPERIMENTAL DETERMINATION OF THE VOLUMETRIC MASS TRANSFER COEFFICIENT. Journal of Chemical Technology and Metallurgy 48, 351-356.

Pegel, A., Reiser, S., Steurenthaler, M., and Klein, S. (2011). Evaluating disposable depth filtration platforms for mAb harvest clarification. BioProcess Int, 9, 52-56.

Proulx, P., Groleau, D., and St-Pierre Lemieux, G. (2019). Introduction on Foam and its Impact in Bioreactors. Canadian Journal of Biotechnology 3, 143-157.

Radoniqi, F., Zhang, H., Bardliving, C.L., Shamlou, P., and Coffman, J. (2018). Computational fluid dynamic modeling of alternating tangential flow filtration for perfusion cell culture. Biotechnol Bioeng 115, 2751-2759.

Wang, S., Godfrey, S., Ravikrishnan, J., Lin, H., Vogel, J., and Coffman, J. (2017). Shear contributions to cell culture performance and product recovery in ATF and TFF perfusion systems. J Biotechnol 246, 52-60.

Wolf, M.K.F., Bielser, J.M., and Morbidelli, M. (2020). Perfuson Cell Culture Process for Biopharmaceuticals (Cambridge: Cambridge University Press).

Zamani, L., Lundqvist, M., Zhang, Y., Aberg, M., Edfors, F., Bidkhori, G., Lindahl, A., Mie, A., Mardinoglu, A., Field, R., et al. (2018). High Cell Density Perfusion Culture has a Maintained Exoproteome and Metabolome. Biotechnol J 13, e1800036.