# A Common Framework for Integrated and Continuous Processing

© 2020 AstraZeneca

Authors

Jon Coffman	 Sevda Deldari	Steve Rose	Ujwal Patel	Jennifer Pollard	jennifer_pollard@merck.com Maen Qadan 	qadan_maen@lilly.com Schaefer, Eugene [JRDUS] ESchaefe@its.jnj.com Joseph Shultz 	joseph.shultz@novartis.com, David.Pollard@sartorius-stedim.com

Irina Ramos	Nicholas Guros	Ken Lee 	Lisa Connell-Crowley 	lisacc@just.bio Brower, Mark 	mark_brower@merck.com Nihal Tugcu 	nihal.tugcu@sanofi.com Pollard, David 	 John Erikson	erickson@udel.edu Dr. David Paolella 	david.n.paolella@gsk.com  Hart, Roger 	rhart@amgen.com Guenter Jagshies, jagschies@icloud.com

# Abstract

There is a growing application of integrated and continuous bioprocessing (ICB) as a manufacturing platform for the production of recombinant protein therapeutics produced from mammalian cells. At first glance, the newly evolved ICB has created a vast diversity of platforms. A closer inspection reveals convergent evolution: nearly all of the major ICB methods have a common framework that could allow manufacturing across a global ecosystem of manufacturers using simple, yet effective, equipment designs. The framework is capable of supporting the manufacturing of most major biopharmaceutical ICB and legacy processes without major changes in the regulatory license.

This paper reviews the ICB that are being used, or are soon to be used, in a GMP manufacturing setting for recombinant protein production from mammalian cells. The adaptation of the various ICB modes to the common ICB framework will be discussed, along with the pros and cons of such adaptation. The equipment used in the common framework is generally described.

An example plant built on the common framework will be discussed. The flexibility of the plant is demonstrated with batches as small as 6kg or as large as 500kg. The yearly output of the plant is as much as 8 tons.

# Introduction

Many biopharmaceutical companies are considering switching from fed-batch bioreactors and independent, batch downstream unit operations to integrated and continuous bioprocessing (ICB), which uses a mammalian host in continuous perfusion bioreactors and a linked and continuous downstream. ICB has well-reviewed cost advantages for clinical [ref]   and, for some scales, commercial [ref]  manufacturing.

The literature describes a dizzying array of options for ICB, so much so that one might think that biopharmaceutical companies would choose different and incompatible implementations.  Here we report and review several of the largest biopharmaceutical companies implementing ICB to show there exists a common denominator in the industry. We further demonstrate that the common framework uses many of the familiar batch processing operations, and allows backward compatibility with legacy batch processes.

The specific choices made for implementing ICB in manufacturing are not often well described in the literature. Only a fraction of published and presented work is implemented. The common framework process is based on the authors’ knowledge of what is being implemented in their company, as well as interviews with other bioprocess engineers across the industry. This article will not disclose the details of individual companies’ platforms except where a citation exists, or disclosure permission is given.  Some features that build on top of the framework will remain proprietary and will not be described.  All of the integrated and continuous bioprocesses will fit into a plant built on the common framework without significant regulatory changes and without large capital investments. The framework will not require new process characterization studies, nor will it require new virus removal or inactivation studies. Regulatory filings and technology transfer would, however, benefit from some foreknowledge of this framework process.

# The Common Framework for Integrated and Continuous Manufacturing

The known ICB being implemented in clinical or commercial manufacturing are shown in Table 1. These processes use a mixture of N-stage perfusion, and some form of multicolumn chromatography with linked processes.  Some processes are integrated end-to-end, from the bioreactor though to the final concentration step. Others pool the product in-process to define a lot.  Some processes run for 14 days, many run for 20-30 days.

The common ICB framework that fits these processes is shown in Figure 1. A common framework is possible because each ICB was derived from a common batch mAb platform processes for monoclonal antibodies [Kelley 2008, etc].

The bioreactor in the ICB framework is run continuously, harvesting the product through a filtration step. The bioreactor links to an integrated downstream that is based on batch chromatography processes. Instead of simulated moving bed chromatography, the framework uses dual column chromatography. In many cases, the chemistry (resins, buffer composition, and membrane composition) is identical to the historical platform. Product pools are replaced with surge tanks. The low pH virus inactivation step is flexible, allowing batch or continuous inactivation. The common framework uses media and buffer concentrates to enable high productivity processes. In-line titration is also required for many linked unit operations. Process Analytical Technologies (PAT) are used, but in this common framework, they are not more complex than those used in batch processes. The common framework also supports fed-batch bioreactor processes with a filtration harvest step.

# N-stage Bioreactor

The framework is enabled by the use of perfusion to achieve high specific productivity in either a steady state or dynamic perfusion.  The industry has many years of commercial  perfusion manufacturing scale  experience at large scale up to 2000 L.   Over 17 commercially launched biologics have used perfusion over the last 25 years using a range of cell retention devices including gravitational settlers, ATF and TFF [lindskog]. The industry has seen cell specific productivities in the range of 20 to 90 pg/cell/day range [Allread][Vijasankaen] with cell densities in the 30-70 Million cells/mL. Typically steady state perfusions has been  achieved at 2- -4 g/L day at 70 mil cells/mL for up to 60-80 days [Brower], [Warikoo], [Ozturk] others to add].  Given this industry experience and, that this framework is designed to be compatible with the future productivities, then its reasonable to assume bioreactor productivity be the target of 4.7 g/L/day (Table 2) with the relatively high cell specific productivity, 40pg/cell/day is achievable. ,This requires a relatively high average cell density of 120Mcells/mL which has been achieved at lab scale [ref] and we expect to be route during the next five years.  Indeed, as discussed below, the only upper limit to the framework process with respect to bioreactor productivity is solution preparation and storage, and the maximum flow rate on the Protein A skid.

(will add two sentences on need for automated bleed control )

We have found that cell lines that have been selected for fed batch adapt easily to perfusion cell culture. Unlike fed-batch, the growth rate is not as critical for perfusion; cell specific growth rate and tolerance to high cell density and low oxygen are more important for perfusion than fed-batch.

The amount of media required for perfusion bioreactors is many-fold higher than that of fed-batch. Perfusion media must be developed that minimizes the volume of media required. Perfusion media can be adapted directly from fed-batch media by mixing a portion of the basal media with the concentrated feed media, removing components unneeded for perfusion, and separating acid, base, and neutral soluble components [Life Technologies 1992]. This media can be from three to five-fold more concentrated than the original fed-batch media. The example below assumes that three-fold media concentrates are used.

Some ICB will not have three-fold media concentrates. The framework process would require either more frequent media preparation during the batch or the use of fewer bioreactors.

# Cell Retention

The harvest for a fed-batch occurs at the end of the batch; the harvest for perfusion occurs continuously. The product is removed from the bioreactor and the cells retained most often by an alternating tangential flow filtration (ATF) system, and sometimes a tangential flow filtration (TFF) system. Typically, ATF systems are used for 500L bioreactors and below; Two or more ATF systems may be used to support a 1000L perfusion bioreactor. Some authors have developed processes that use as many as four ATFs to support a 2000L perfusion bioreactor. They take up considerably more floor space than the TFF systems, and must be immediately adjacent to the bioreactor. ATFs require less development time than TFF systems, and thus may be used in early phase clinical trials or commercial products that do not have high material needs.

Some companies have transitioned to TFF systems at 1000L bioreactors and above to avoid the need for multiple ATF systems. A TFF system is slightly cheaper than ATF systems, and can be scaled more easily.  For the 2000L scale, the capital cost for a TFF system is therefore less than ¼ that of the ATF systems.

ATF and TFF filters can also be back flushed with perfusate or fresh media periodically [ref needed]. This backflushing would require a pump separate from the filtrate line. Back flushing can extend the life of the filter significantly, though we are aware of no published studies demonstrating this.

TFF systems suffer from Starling flow [Radoniqi 2018, others], which causes poor filter utilization and faster fouling. This fouling causes increased product retention [Wang, et al, other references]. ATF systems use Starling flow to clean the membrane during each alternating cycle. As a result, ATF systems have better product retention profiles and possibly larger filtration capacities.

Starling flow can be mitigated in many ways, the easiest perhaps by the use of shorter membranes. Achieving the same filter area with shorter membrane length could require a large number of hollow fibers, and a concomitant high retentate flow rate.  Or, short membrane modules could be installed in series, each with a separate permeate pump, as shown in Figure 2. The method of using stacked membrane modules each with a separate permeate pump is a relatively simple way of scaling up the TFF step. The shear encountered between modules must be minimized in the coupling design. Such a TFF design could enable perfusion up to 12kL bioreactors.

The common framework would use the same permeate pumps for both the ATF and TFF set up.  The retentate pumps and associated controllers would be unique to either the ATF or TFF set up.

# Dual-column chromatography

Multicolumn chromatography has been used in many of the authors’ downstream processes (Zydney 2016). [ref] The simplest common framework for these chromatography systems is two columns. Although many processes use more than two columns, the multicolumn nature is not used in any step but the load step: The systems use two columns during the load step to overload one column while capturing the product in the flow through onto the second column. [ref] All other steps are effectively single-column operations. Thus, the simplest embodiment of the multicolumn chromatography skid is a two-column system. A two-column system has about 1/4 fewer valves and ½ the number of pumps as a three-column system. This simplicity reduces the cost of the system and allows operators familiar with batch chromatography to easily operate the system.

Rather than use a bespoke two-column system, one could use single column systems side-by-side. Many single column systems are less than half the cost of a multicolumn system. Each single column system could, in addition, be used separately, increasing the flexibility of the plant.

The dual column system can be used as a replacement for one or more columns without impacting the regulatory process description. More columns do, however, increase the apparent continuity of the product flow into and out of the step. Also, above a certain titer, the two-column process cannot be continuous because the load step is completed faster than the subsequent wash, elution, regeneration steps. A two-column process therefore requires a surge tank for the load when the titer exceeds a threshold.

In-process pool stability is a key consideration for integrated processes. The process should be integrated and continuous at least up until a stable process intermediate is obtained. At this stable intermediate, a pool of the entire bioreactor run can be made and held. This pool can be defined as the lot.

[maybe to be deleted for brevity] Bioburden control is a critical factor for the capture step. The step is hydraulically linked to the nominally sterile bioreactor. Media conducive to bioburden growth flows through the column for many hours during the load. Bioburden in the chromatography resin can grow quickly. There have been significant efforts to nominally sterilize the Protein A resin with gamma irradiation, caustic solutions, and oxidative solutions such as peracetic acid [ref]. There exists today no commercially available Protein A chromatographic column that is validated to be sterile, and thus the potential for bioburden to enter the process through the Protein A resin must be considered. Many of the authors have successfully designed bioburden control strategies that use standard processes, and that do not require nominally sterile Protein A columns. Such bioburden control strategies are the subject of another paper.

The Protein A step requires a titer measurement periodically to calculate the load volume and flow rate. For dynamic perfusion, this titer may need to be measured every twelve hours. For constant-cell density processes, this titer may be required every few days. Process Analytical Technologies (PAT)exist to measure titer more frequently. These technologies are not part of the common framework. Manual sampling with at-line testing and less than one hour turn around is sufficient. Faster testing may drive higher resin utilization and efficiencies.

The solution and water utilization of the downstream is a major consideration, and in many cases the main bottle neck in the ICB downstream. The standard productivity measure used in the literature for this step, mass product per volume resin per unit time, [ref] is not the most relevant to manufacturing because the resin is almost never the limiting factor. Instead, one should consider a product mass index, or the solution mass per mass of product produced. This measure is the nearly same as the measure used to compare the environmental efficiency of processes, the Product Mass Index (PMI), which would include all raw materials and consumables in the numerator.

The use of solution concentrates is essential to maximizing the productivity of ICB. Floor space for production would be mostly solution preparation and solution storage without buffer concentrates. Even with five-fold concentrates, many authors have seen that the solution preparation and storge area is larger than the main production area. The volume and number of solutions drives the size, cost, complexity, and throughput of high-capacity ICB manufacturing facilities. These considerations also benefit classic batch processing, though not to the same degree.

The volume of solutions required scales inversely with the capacity of each column. An example based on common operational parameters is shown in Table 3. The example provided uses standard resin capacities and wash strategies, including 14 solutions for the entire downstream, with the exception of titration. Thus, the plant requires at least 14 large solution hold tanks. For each kg of drug substance produced, about 29L of each 5x buffer concentrate is required, or a total of 400L of buffer concentrates per kg of material produced (Table 3). This PMI compares well with 2000 L of 1x buffer per kg for the two-column platform process (Kelley 2008).

The same solution should be used across multiple steps. In this example, the only repeated solution is the sanitization solution. Other solutions that can be reused are the equilibration for the Protein A, the anion exchange, the VRF, and the UFDF. Also, the high salt regeneration solution can be reused across the ion exchange steps. This strategy does not reduce the total volume of solutions. It does, however, allow simpler logistical solution preparation and storage.

# Low pH virus inactivation

The low pH inactivation step typically follows the Protein A step. The step can be operated in either periodic batch mode or with a continuous plug flow reactor. The framework process can satisfy both modes with in-line titration of the product stream. For typical product concentrations, the acid solution flow rate is proportional to the product concentration [ref Pfizer/BI patent application].

For the systems that use continuous inactivation, a plug-flow reactor (PFR) is required. Many PFG designs have been published for low pH virus inactivation. Since the amount of acid and base changes with product concentration, the stream might require dilution with water to maintain constant ionic strength. A schematic of a skid that accounts for these control strategies is shown in Figure 3. The subsequent chromatography step must be developed accounting for variable load flow rates.

Better pH measurements would be beneficial for this step. pH probes that do not require daily calibration, or can be calibrated automatically in closed systems, would be beneficial. pH probes that can withstand gamma irradiation would also be beneficial.

# Virus Reduction Filter

Earlier investigations about mechanistic understanding of virus filtration have reported the impact of factors such as variability in product concentration, pH, ionic strength of the feed and fluctuations in operating pressure (Bakhshayeshi and Zydney, 2008; Bohonak and Zydney, 2005; Bolton et al., 2005; Ireland et al., 2005; Rathore et al., 2014; Strauss et al., 2017; Syedain et al., 2006; Tateishi et al., 2001; Wickramasinghe et al., 2010). A complex interplay between these factors can impact the overall performance of the VRF process by eventual flux decay and accidental virus breakthrough in the filtrate.  Some processes are able to avoid this flux decay and operate essentially continuously for long periods, enabling end-to-end processing.  Other processes require operation for less than a day, essentially in batch-wise operation.

The filter capacity is often based on the volume passing through the filter. Therefore, the filter area used is the same independent of where the lot is pooled. The flow rates for each mode are quite different. The continuous end-to-end process has about 1/20th to 1/60th the flow rate of the entire lot of material at once on the VRF. The framework process pools material before the VRF for small lot sizes (such as those needed in early phase clinical trials), or changes the virus filter daily for large material requirements. A single use skid with a high-flow and low-flow tubing set and pumps would allow the processing of both modes of VRF.

The continuous process may suffer from decreased virus removal factors due to transport mechanisms that act over days [ref], or by periodic flow pauses. The choice of filter is critical to the successful operation of the continuous VRF.

# Concentration and diafiltration Steps

Many processes use the same ultrafiltration and diafiltration (UFDF) set up as for batch. Processes that diafilter the entire lot of material at once will suffer from the need for very large pumps and filters. This strategy is not recommended.

The framework process can be configured to allow batch-wise UFDF or single-pass, continuous UFDF. The step uses the same pumps and similar filter areas, while the flow path is redefined to allow both modes of operation.

The batch-wise process is shown in Figure 5A. First, a small UF system concentrates the VRF stream to a desired level. Periodically, the concentrated material is moved to a small diafiltration system. After diafiltration, the sub-lot is pooled with other sub-lots. The larger pool is concentrated with a small UF system over the course of the run, allowing fine control of the final pool concentration.

The single-pass, continuous UFDF process uses SPTFF (Jungbauer, 2013). to continuously concentrate the product. The product then passes through an in-line diafiltration (ILDF) step made up of several stages of dilution and SPTFF modules. These modules are most efficient when operated in a counter-current mode (Arnold et al., 2019; Huter and Strube, 2019; Jabra et al., 2019; Nambiar et al., 2018; Rucker-Pezzini et al., 201). The final drug substance concentration is achieved through a final SPTFF step. A schematic of such a process is shown in Figure 5B.  Using counter current DF can reduce the diafiltration buffer volume by roughly a factor of two, mostly by allowing a higher concentration of product during the diafiltration operation.

The membrane sizes for the batch-wise and continuous modes of operation are similar, on the order of 1m2. Smaller lots are accommodated by doing fewer sub-batches.

Material is pooled during the entire lot. As with fed-batch processes, the product quality from the bioreactor may vary with time. The control of the product quality as material is pooled requires a sophisticated GMP and regulatory strategy.

# Implementation of clinical and a commercial 8-ton per year ICB

A large-scale staged implementation of the framework is shown in Figure 4. Stage one of the implementation uses a 500L SUB. It is capable of producing as small as a 0.5kg batch, and may be considered representative of a clinical trial manufacturing facility. For many products, 500L is the commercial scale ICB. The plant is capable of producing up to 30kg batches or about 0.5 tons per year.

Some products, however, need higher levels of production. The framework plant can expand by adding three 500L bioreactors in Stage Two, as well as more media and solution preparation and storage containers shown in Figure 4B. One remarkable feature of the common ICB framework is that the downstream remains unchanged, as discussed in the accompanying article (Coffman 2021). The “numbering up” of bioreactors does not change the regulatory filing. This Stage Two plant can produce up to 130kg batches and 2 tons per year (Table XXX).

The plant can expand further in Stage Three by adding more 500L bioreactors, or using the existing 500L as N-1 perfusion seed bioreactors to inoculate between one and four 2000L SUBs, as shown in Figure 4C.

More media and solution preparation and storage are also added. Remarkably, the downstream equipment remains the same [Coffman 2021]. Moving a product from 500L to 2000L bioreactors has a moderate regulatory impact and requires agency notification and comparability studies. This plant can produce 500kg batches in 20 days and up to 8 tons per year. The capacity is limited by the solution and media preparation and storage, and, to a lesser extent, the maximum flow rate of the capture step [see Coffman 2021]. The plant can operate either the 500L or the 2000L SUB as the N-stage production bioreactor to allow legacy production. The plant also supports legacy fed-batch production as well, with some restrictions on the harvest step.

As shown in Figure 4C, the 500kg batch requires only 80,000L of media concentrates, which can easily fit into 16 x 5,000L tanks. This tank size is suggested because it is the largest single-use container available today. The 500kg lot would require about 24 x 5000L tanks for the downstream, which is a manageable number.  Media and solution preparation and storage is therefore not a major consideration for the framework process in new facilities.  It is a major consideration for retrofitting legacy facilities.

Water production is also a consideration. The total amount of water used to make up and dilute the media and solution concentrates over the course of 20 days is 1 ML, or about 50,000 L per day. This volume is not large for large scale facilities. The largest rate of water production would be required for the initial media and solution preparation, which can easily be spread out over the duration of the run. Thus, the water consumption rate for media preparation is not a major factor in this 4x2000L design for new facilities. It may be for retrofitting facilities.

The capital required for such a plant is relatively small. Jagshies correlation  [NAS, 2020], based on installed bioreactor volume, estimates the cost of the plant to be about $100M. Using a ten-year depreciation schedule, the capital depreciation would contribute about $1/g to the COGM.

An 8-ton plant built on ICB and with single use bioreactors is unprecedented. It rivals the largest fed-batch suites. Yet all the equipment necessary can be built from existing components. As an example of the power of ICB, the highest flow rate in this 8-ton plant is an astonishingly small 9 L/min. The volume of bioreactor(s) that the common framework can support is limited by media and solution storage, and the maximum flow rate of the capture step. In these examples, the maximum flow rate was set to 11L/min arbitrarily to yield a 500kg batch. Downstream skids with maximum flow rates of 50 L/min are common in large stainless-steel plants. If such equipment were used, the downstream could support a total bioreactor volume of 48,000L at 1.5vvm, and yield a lot of over 2.5 tons in twenty days. While no product today has such requirements, therapies for wide-spread diseases like Alzheimer’s and COVID-19 may require massive amount of material [ref]. Unfortunately, these treatments have a low probability of technical and regulatory success. The common framework for ICB allows a staged investment in new facilities, and can grow with market demands. This flexibility cannot be found in standard batch processes.

Sustainability Framework

The integrated continuous process can take advantage of intensification, single use and multi column chromatography to show the lowest process mass intensity for mAb production.   Given the water intensity of biologics production [Budzinski] it's no surprise that replacing stainless steel with single use reduces water (used for cleaning) by 70% and lowers the PMI by 60%.  Further PMI reductions, to values around 2000 (Table 3) are achieved by the continuous framework, by the combination of factors. These include intensified processing (perfusion integrated to  multi column chrom), equipment integration with single use closed processing reducing cleaning water, and buffer reduction (10-15%) from the multiple column chromatography operation [Brower]. THe electrical energy consumption from HVAC is a dominating factor in the life cycle assessment of bioprocesses [Pietrzykowski] and is reduced by the continuous framework.   The intensified and integrated processes enable upto a 50% smaller facility footprint . The use of fully closed single use integrated processing can support the ball room classification grade reduction from C to D and provide an electrical energy reduction by 15-20%. Further improvements  to the PMI are expected with the reduction in plastics and packaging waste by improved reuse and recycling that will support a circular economy in the coming years [Barbaroux]