# The Design Basis for the Common Framework for Integrated and Continuous Bioprocessing

© 2020 AstraZeneca

Jon Coffman, Rick Lu, Steve Rose, Ujwal Patil (AstraZeneca)

# Abstract

An 8-ton per year manufacturing facility is described based on the framework for integrated and continuous bioprocessing (ICB) common to all known biopharmaceutical implementations. While the output of this plant rivals some of the largest fed-batch plants in the world, the equipment inside the plant is relatively small: the plant consists of four 2,000L single use bioreactors and has a maximum flow rate of 13 L/min.  The equipment and facility for the ICB framework is described in sufficient detail to allow biopharmaceutical companies, vendors, contract manufacturers to build or buy their own systems. The design will allow the creation of a global ICB ecosystem that will transform biopharmaceutical manufacturing.

The design is fully backwards compatible with legacy fed-batch processes.  A clinical production scale is described that can produce smaller batch sizes with the same equipment as that used at the commercial scale. The design described allows the production of as little as 100g/day to nearly 40kg per day.

# Introduction

Many biopharmaceutical companies are investigating the next generation of manufacturing by developing integrated and continuous bioprocessing (ICB).  A seen in the companion article, a common framework for ICB exists that would enable most, if not all, ICB processes currently being implemented. It can be implemented in a staged approach in new facilities.

The plant and equipment designs described here can produce quantities of recombinant proteins that would satisfy any market need.   Very few products require over 1 ton of material per year (Kelley 2009); the framework design is capable of 8 tons per year.

The examples include a 500L bioreactor with a low productivity cell line and 2000L bioreactors with a high productivity cell line. The 500L bioreactor makes about 0.5kg of material in the bioreactor per day, which results after 20 days in a 6kg batch. The 2000L bioreactor makes about 10kg of material per day, which results after 20 days in a 130kg batch. A four 2000L process is described that results in as much as a 500kg batch over 20 days, or about 8 tons per year in 17 batches.

# Materials and Methods

The bioreactor design was developed based on common and well-known engineering principles. Assumptions are listed in Table 1. The cell-specific oxygen rate was estimated to be 0.2-0.3 pmol/cell/hr [1999 Jorjani & Ozturk]. The required kLa assumes the dissolved oxygen (DO) setpoint is 50% saturation, and that pure O2 is sparged into the bioreactor. The measurement of kLa is the dynamic gassing out method used by Moutafchieva et al (2013). The upper limit on gas flow rate is assumed to be 0.2vvm, due to marginal gains in effective gas transfer above this flow rate, i.e. jetting. The operation and design of the impeller(s) is not specified due to the wide variety of viable options  from many vendors, but it is important for the impeller design operate over a range of power inputs without risking damage to the cells via shear.

The media was made up of three concentrates, 5x, 12x, and 20x, that when mixed, create a 3x concentrate. Water diluted the media during perfusion.

The cell retention system was designed as a TFF or an ATF with a membrane capacity of 1000 m2, which is a conservative value [Wang 2016]. The TFF system was assumed to be a 1mm hollow fiber operated with 2000 s-1 shear [ref] but could have easily been a flat sheet membrane. The overall TFF membrane configuration requires multiple 40 or 50cm hollow fiber modules stacked in series (Figure 2). Each module has a separate permeate pump to minimize Starling flow between modules. The TFF and ATF modules were assumed to be changed once per run.

The dual column chromatography steps were designed as with the assumption shown in Table 1. The capacity of the columns was assumed to be 20% greater if they were operated in series during part of the load, to allow the columns to achieve near-equilibrium capacity without impacting yield.  The systems are shown schematically in Figure 3. They have nine inlets and three pumps. One pump and seven inlets are used for solution concentrates. One inlet and pump is used for load material. One inlet and pump is used for water to dilute. The flow rate range of the buffer concentrates is between 0.3 and 3 L/min. The flow rate range of the water pump was 1 to 10 L/min. These ranges were chosen to allow maximum accuracy of the solution dilution. The accuracy of the solution concentrate pump should be about 1%, which would allow a 5% error in solution concentration after dilution. The flow rate range of the load pump was between 0.5-10L/min, which is a wider range since the load flow rate to the capture step does not require the same level of accuracy as the solution dilution.

The steps were designed with three column volumes per solution. For the bind/elute columns, the solutions were equilibration, wash 1, wash 2 (Protein A only), wash 3 (Protein A only), elution, strip, and regeneration/sanitization. The flow through step had two equilibration steps, wash, strip, and regeneration/sanitization. The chromatographic solutions were presumed to be concentrated at 5x the normal solution concentration, to be diluted in line by volume, with no pH or conductivity feedback control loop.

The solution velocity through the columns was set to 300 cm/hr. If the dual column chromatography was used, the bed height was set to 10cm. If two columns were not set in series during the load, then the bed height was 20 cm.

The minimum capacity for the Protein A column was set to 10g/L. The maximum for single column chromatography was 60g/L. The minimum number of cycles per day was one; the maximum on the Protein A for the two-column set up was 16 per column for the bioreactor scenarios with a productivity of 4.8g/L/day. The average number of cycles per day was 10 per column, so that the full 20-day batch had 200 cycles. The number of cycles depended upon the load time for load concentrations lower than 3.3g/L for the dual-column setup, or bioreactor productivity less than 5g/L/day.

The virus inactivation requires a volume acid and base in approximate proportion to the volume of the peak (Figure 4). These volumes were assumed to be 10% each. These solutions were not diluted.

The virus reduction filtration (VRF) step was operated on a daily basis for the commercial scale. It was run twice per lot for the clinical scale example. Due to the lower limit of flux allowable to some filters, this daily operation may only require a few hours of processing. Surge tanks were used before and after the VRF. The capacity of the filter was assumed to be a 700 L/m2 which is below the preferred commercial average. The flush was 75 L/m2 before and after the filtration.

The final concentration and diafiltration step were designed to support both batch UFDF operation and single pass tangential flow filtration (SPTFF) operation. The batch operation used three batch operations run in series.  Each were operated 22 hours per day, to allow sanitization between days if needed. The retentate flow rate was set to 390 L/m2/hr, a typical value. The permeate flux was set to 45L/m2/hr, which is also typical, though it will vary from step to step. The filter size for the concentration steps was 0.1m2 to 0.3m2. The DF step membrane size was 0.8-3.3 m2. The first and last retentate pump, as well as the DF solution feed and VRF product feed, required pumps with a range between 0.1-3L/min. The DF system required a retentate pump with a maximum flow rate of 30L/min.

The product, in this case assumed to be a well-behaved mAb, was concentrated to 50g/L, diafiltered against six diavolumes of solution, and then concentrated to 150 g/L to allow a 100g/L target after adding a spiking solution. The flushing volume and spiking volume were not estimated.

The single-pass based system included an initial single-pass TFF (SPTFF), a counter current diafiltration (CC-DF) system, and a final single pass TFF. The initial SP-TFF concentrated the product to 150g/L, a relatively high value [ref]. It assumed a filtration flux of 10L/m2/hr [ref]. The filter sizes ranged between 0.7-2.8m2. The CC-DF used three stages with countercurrent diafiltration buffer flow. The filtration flux was assumed to be 15m2. The ratio of DF solution flow to product flow was set to be 11.

The process was operated for 20 days, in a plant that operated for all but two weeks per year. One day was assumed for changeover.

# Results and Discussion

The high-level consequences of the common framework ICB are discussed in the accompanying article (Coffman, 2021). Here we discuss the details of the design to demonstrate the flexibility and productivity of the framework ICB. The examples used are shown in Figure 1. They are described in more detail below.

## Bioreactor Design

Constant cell density, or “steady-state,” perfusion typically requires controlling the cell density by bleeding cells at the same rate as the growth rate of the cells (Wolf, Bielser et al. 2020)[ref]. These perfusion systems typically operate between 50 and 75Mcells/mL. Dynamic, or non-steady-state perfusion, where the cell density is not directly controlled by a bleed, can reach cell densities of 100-150Mcells/mL (Wolf, Bielser et al. 2020). These cell densities would require oxygen transfer rates between 15 and 50 mM/hour (Jorjani and Ozturk 1999). The gas/liquid transfer coefficient kLa  between 15 and 70 hour-1 would be required￼￼.  Today, there are a few examples of perfusion cell culture at 240Mcells/mL￼￼. If the industry is to expand to this cell density, a kLa of over 100 hr-1, or lower cell-specific oxygen uptake rates would be required.

The amount of material made in a process depends upon the cell line, which is often the same cell line used in fed-batch processing. In these scenarios, the cell specific productivity, Qp, is assumed to be 20 and 40pg/cell/day, though productivities as high as 90pg/cell/day have been reported (Li, Shen et al. 2010, Li, Vijayasankaran et al. 2010, Alldread, Birch et al. 2014, Wolf, Bielser et al. 2020). The Qp range represents low to moderate productivity monoclonal antibody-producing cell (Jostock 2011, Wolf, Bielser et al. 2020).

The gassing strategy for a perfusion culture is a three-fold balancing act between control of dissolved oxygen (DO), pCO2 accumulation (i.e. pH and subsequent base utilization), and foam formation. Achieving desired control in any one aspect can result in an undesirable result in another aspect, as is subsequently presented. Principally,

the supply of oxygen to the cells can easily be the limiting factor inachieving high cell densities (Ozturk, 1996, Zhu, Marie et al. 2017). To meet this higher oxygen demand there are two aspects of gassing that need to be considered: mass-flow controller capacity and sparger design. Generally, mass-flow controllers delivering air and O2 for perfusion processes requires significantly higher flow capacity than fed-batch processes, ranging from 0.05vvm to 0.2vvm. s . With respect to sparger design, micro-spargers have been demonstrated toimprove the oxygen transport capacity needed to achieve a desired DO at perfusion-scale cell densities.However, microspargers raise several concerns, including their potential to cause cell damage due to shear when the microscopic bubbles burst (Wolf, Bielser et al. 2020). Microspargers have also been observedto cause higher pCO2 accumulation than traditional macro, i.e. ring or drilled-hole, spargers, which lowers culture pH, and subsequently more base utilization which can be increase cell stress and the rate of cellular apoptosis. Additionally, microsparging creates a thicker layer of foam than traditional spargers which requires enhanced methods of foam control to manage, and is a concern with respect to fouling bioreactor exhaust.

A  dual-sparger design that is composed of a micro and a macro sparger permits the most flexibility in configuring the gas flows to balance the requirements for dissolved oxygen, dissolved carbon dioxide, and foam management in a perfusion bioreactor. In this design, the microsparger component generally provides the required dissolved oxygen to a culture via relatively smaller bubbles and the macrosparger component generally provides a means to manage pCO2 accumulation via stripping carbon dioxide from the system with relatively larger air bubbles. The dual-sparger design can also provide an additional benefit of reducing the amount of foam generated via destruction interactions between the two bubble sizes (Karakashev, Stoyan et al. 2012).

Even with a dual sparger design, foam and aerosol management is more difficult in perfusion cultures due to relatively higher agitation and gas flow rates than fed-batch cultures. The first line of defense is to manage foam generation in the bioreactor itself through means such as mechanical disturbance (ref), adjustment of overlay humidity (ref), and automated addition of antifoaming agents (e.g. simethicone, ref) as a function of the rate of foam generation and foam thickness, which increases proportionally to protein concentration. Redundant filters and/or vent traps are essential components of the exhaust manifold to manage the collection of foam and aerosols, as is the ability to seamlessly switch between such unit operations while maintaining a sterile boundary. Advanced depth filters (ref) and condensate systems (ref) demonstrate an enhanced ability to manage aerosol generation, and should be considered for implementation in any bioreactor exhaust manifold.

The last consideration for the perfusion bioreactor gassing strategy is the control strategy. Essential to maintaining the balance between DO, pCO2 accumulation, and foam generation is establishing and bioreactor-specific relationships for the parameters that govern this balance, including the gas transfer (i.e. kLa) for a given gas species and sparger design. Therefore, it is essential for perfusion bioreactor control architecture to permit the use of reasonably complex algorithms to manage these three control elements, as well as the ability to incorporate so-called offline, at-line, and online measurements such as cell density, temperature, lactic acid concentration, glucose concentration, and pH.

(Wolf, Bielser et al. 2020) [ref].

.

Standard perfusion SUB assemblies should include 1” vent lines with weldable tubing. (not sure where this is from?)

These bioreactors may become exothermic and require cooling due to the exceptional quantity of cell mass, a novel concern for perfusion as compared to batch mammalian cell culture. Cells generate about 28pW/cell (Kemp and Guan 1997) or between 1.4 to 4.2 W/L for cell densities between 50M-150 Mcells/mL. A 2000L vessel would generate between 3000W to 8000W, which is easily handled by cooling jackets. Cooling a 12kL bioreactor may, however, become a limitation as the cells could generate up to 50kW. The heat capacity of the room temperature feed solution and gasses, as well as evaporation into the sparge gases, provide negligible cooling.  A heat exchanger in line with the tangential flow filtration system may become necessary.

The perfusion rate is 1.5 vvd (Wolf, Bielser et al. 2020). These perfusion rates may be spread across one to three media concentrates with 5x to 20x concentration factors [Ref Hiller, IBC 2016, Lin IBC 2016]. In the scenarios shown in Table 1, the media concentrates are made up of a 5x, 12x, and 20x concentrate, which, taken as a whole, are effectively a 3x concentrate. Multiple media feed pumps sized from 0.02 vvd to 0.3vvd would be needed. For a 2000L bioreactor, this is a surprisingly small 20-400mL/min. One or two larger pumps for the diluent are needed to supply the remaining volume of about 1vvd, or 1400mL/min for a 2,000L bioreactor. A glucose concentrate and antifoam will also need to be added with even smaller flow rates. The pump rates should be capable of being set by complicated calculations that may rely on PAT or on mechanistic models. The pump rates should be capable of being updated on an hourly basis.  Overall bioreactor level control is required to control the feeds or the permeate rate.

The media concentrates may be added through a single line with a static mixer. Typically, a line of water enters the static mixer, and a series of T-connections allow the mixing of each concentrate in a serial fashion. The order of addition of each liquid concentrate may need to be determined, as some concentrates may cause precipitation if mixed in the wrong order.

The use of media concentrates greatly reduces the size of the media preparation and media storage area. In the 4x2000L scenario shown in Tables 1 and 2, the media can be stored in sixteen 5,000L single use containers. The 20x media would require an additional 2,000L container.

A design that might be suitable for a clinical manufacturing facility is shown in Column 1 in Tables 1 and 2. The 500L bioreactor operates at 50Mc/mL with a 20 pg/c/day cell line. This scenario represents one that can be achieved by converting a fed-batch process over to perfusion cell-culture without much difficulty. It also represents one that might occur early in development, when process optimization and intensification has not occurred. The bioreactor makes 500g/day.

A commercial bioreactor set up is shown in Table 1 and 2, and column 3, operating at an average cell density of 120Mc/mL with a 40pcd cell line. In this scenario, four 2,000L bioreactors are operated to feed a single downstream. The bioreactors together produce 38kg per day.  Over the course of the 20 day run, over 500kg of material is pooled into a single lot.

## Cell Retention Design

The TFF design is shown in Table 1 and 2 for a 500L and 2000L bioreactor.  While the design presumes a hollow fiber filter, flat sheet filters will also work. The membrane capacity is assumed to be 1000L/m2, which is conservative compared to some published in the literature (Wang, Godfrey et al. 2017). A typical 1mm fiber lumen diameter was assumed, along with typical shear rate of 2000 s-1. In order to mitigate against Starling flow, the 500L TFF was designed with one 41.5 cm long hollow fiber modules. The 2000L bioreactor was design with three or four hollow fiber modules in series (see Figure 2, A). Each module had a separate permeate pump. In order to maintain reasonable cross flow rates of less than 90 L/min, the design assumed that the membrane module would be changed one time during the run.  This flow rate is well within the range of existing large low shear centrifugal pumps. Without these strategies, the retentate flow rate would be between 500 and 1000L/min, or 0.25-0.5vvm. These flow rates would require very large pumps, and the cells would have 5-10 times more pump passes. Damage to cells through pumps is an important factor to mitigate (Wang, Godfrey et al. 2017).

TFF systems suffer from Starling flow (Radoniqi, Zhang et al. 2018) which causes poor filter utilization and faster fouling. Starling flow can be mitigated in many ways, the easiest perhaps by the use of shorter membranes. Achieving the same filter area with shorter membrane length could require a large number of hollow fibers, and a concomitant high retentate flow rate (Figure 2B).  Or, short membrane modules could be installed in series, each with a separate permeate pump, as shown in Figure 2. The method of using stacked membrane modules each with a separate permeate pump is a relatively simple way of scaling up the TFF step. The shear encountered between modules must be minimized in the coupling design. Such a TFF design could enable perfusion up to 12kL bioreactors.

Switching the filter for the 2000L bioreactor design is best accomplished by the use of Y-connectors and valves between the two sets of filter modules. Manual switching between the two sets of filter modules is likely acceptable for most operations, even in commercial space.

The design calls for one ATF10 systems for the 500L bioreactor and three per 2000L bioreactor. While this takes up a considerable amount of floor space around the bioreactor, it is not a major inconvenience.

The area required for both ATF and TFF on the 2000L bioreactor is 30m2. Modules are commercially available.  The cross-flow rate is 70L/min, which can be handled by an off-the-shelf low shear pump. The ATF and TFF systems require three or four small permeate pumps with a maximum flow rate of less than 500 mL/min. The total flow rate onto the capture step is small, about 8L/min for the 4x2000L design.

Single-use bioreactors require 1.5-inch ports to allow unrestricted flow for each pump system. The 2000L bioreactor requires four ports to support the ATF and two ports to support the TFF operation.

## Dual Column Chromatography Design

Dual chromatography is the simplest and perhaps easiest form of ICB that can be implemented. The high-level skid design shown in Figure 3 allows flexibility for ICB and batch processing. The system uses two pump sizes. A smaller pump is used for the solution concentrates. Larger pumps are used for the diluent and the load.  This design has a large range productivity. The overall accurate flow rate range is 0.3 to 12.5 L/min. High productivity processes use solution concentrates and dilute with water. The largest processing possible on this system is two 25L columns operated in series, allowing for 57.6kg/day. [2-minute residence time, 20 cm bed depth, 300 cm/hr, 16 cycles per day per column]. Two columns operated in parallel can process 48kg/day, due to the decreased effective capacity of the column. The smallest mass that can be processed uses only the smaller pump with no dilution at the lowest accurate flow rate, allowing the use of a 0.6L column. This set up processes as little as 36g per day.  For plants that desire this degree of flexibility, the product flow path on such a skid would be single use, and allow a high-flow and a low-flow tubing kit to avoid product dilution at the low flow rate range.  Most plants would not need this degree of flexibility and the full range of productivity will depend upon the tubing diameter and the extra-column volume.

The number of cycles per day is set to avoid changing columns during the 20 day run for commercial processes. Each column is cycled 10 times per day, for a total cycling of 200 cycles per lot. While column lifetime may vary from product to product, targeting 200 cycles is not overly burdensome for commercial processes. Poorer column lifetime will require larger columns and fewer cycles per day. If the Protein A resin costs $15,000L/L, then the contribution to COGM is about $1.7/g, which is still one of the highest single contributions to COGM.

The downstream process is tolerant to changes in productivity in the upstream in two ways. Firstly, the columns can be underloaded. The load range the Protein A is assumed to be 10g/L-72g/L. The second is that the columns can be cycled as few as once per day, to as many as sixteen times per day. The lower limit is set, in part, by the bioburden control strategy, in that we prefer to load a column less than 12 hours before sanitizing it. The total in-batch range of productivity is therefore 230 fold. Between batches, the columns can be changed in size from 0.6L to 25L. Thus, the overall batch-to-batch productivity range for the downstream is 9600-fold range.

The system also has UV absorbance detection with two path lengths to allow both high and low concentration measurement. Ideally, the UV detector would be linear up to concentrations of 150 g/L, or about 200 AU/cm. A detector with 0.1mm and 1.0mm path lengths would be sufficient. These detection limits are necessary to enable accurate yield calculations on every cycle. Optionally, the detector would allow a scan of absorbance from 230 nm to 320 nm. Spectral analysis of column peaks can be used to estimate purity [Jorgen Hubbuch]. Additional data input ports should be available for expanding PAT methods that are not described here.

The dual column chromatography skid may not benefit from being single use, depending upon the application. Small lot sizes will have an increased burden of on the COGM of the single use assemblies. This burden is less important in clinical manufacturing, where flexibility and change over time might be more important. This burden is also less important for very large batches, such as those derived from the 4x2000L example. Plants producing more material than the 4x2000L example would be limited by the available SU column sizes, large SU valves, and the number of SU containers for media and solution preparation and storage. SU pumps are not limiting:  A system based on commonly used single use pumps (Quatroflo 4400SU, 83L/min max, and Quatroflo 1200SU, 20L/min max) could conceivably be used to process nearly 400kg/day. The list price for 5,000L single use containers (ThermoFisher, Logan Utah) is less than $2000, which contributes about $0.2 per g of material produced in this scenario where the PMI for media and buffers is 560 L/kg. When 1x media concentrates are used, and the bioreactor productivity is 1g/L/day, the PMI for solutions and media is 2700 L/kg, and the cost of the media storage rises to about $1/g. This cost may become prohibitively expensive for COGM sensitive products.

## Virus Inactivation

The framework virus inactivation step supports both batch virus inactivation and continuous. The skid performs on-line titration of the peak as it elutes from the Protein A step. The acidified peak then goes into a tank for the batch operation or into a plug flow reactor (PFR) for continuous inactivation. At the end of the incubation time, the product stream is neutralized before passing to the next step.

Those processes that use a surge tank prior to the virus inactivation step will have a homogeneous product stream entering and exiting the PFR. This homogeneity allows for either a volume-based titration, or one based on a closed-loop pH control. Those steps that titrate the peak directly will require some feed-forward control based on UV. The UV signal from the Protein A skid will require integration with the virus inactivation skid, or the VI skid will require a UV sensor on the inlet.

## Depth Filtration

High-density fed-batch processes precipitate HCP and other impurities after the low pH virus inactivation process. The particulates are removed typically by a depth filter.

Perfusion based processes do not precipitate with the same frequency or degree. The framework process requires the capability of adding a depth filter. The depth filter has challenges for continuous processing. There are not many depth filters that arrive sterile or have a low bioburden specification. This means that depth filtration is an opportunity for bioburden to enter the process. Some depth filters can be sanitized with various solutions, hydroxide being the most typical.

Since the depth filter capacity is so low (100 L/m2), it will require frequent changing. The framework presumes daily changing.  The filter area is proportional to the Protein A column volume, and varies from 0.5 m2  for the clinical process to 13m2  for the 500kg process. While 13m2 is seen commonly in manufacturing plants, it is large and is best flushed with its own pump system.

## Virus retentive filtration (VRF)

Virus filtration provides a robust and scalable solution for viral clearance in biologic manufacturing. The choice of a suitable virus filter is governed by a combination of feed characteristics and process parameters. Therefore, the lack of in-depth understanding has, for many, restricted the VRF process to a batch operation. Table 1 and 2 depict various scenarios for a VRF process following the second polishing step. As shown in Table 2, the low productivity scenario would necessitate a membrane area of 0.5 m2. Each VRF sub-batch is expected to be operated at 24 h intervals. Here, each sub-batch will take roughly three hours, yielding the final product pool of  277 Litres at 24 g/L (Table 2). The concomitant adjustments in membrane area and other operating parameters towards changes in capacity and productivity are mentioned in the supplementary information (Table 2 & SI Table).

## Ultrafiltration and Diafiltration (UF/DF)

The common framework process enables both batch and single-pass (SP) ultrafiltration and diafiltration (UFDF). The framework uses a flexible single-use skid with the same pumps and different flow path to accommodate both operations.

The batch UF/DF step is shown in Figure 5  and involves batch tangential flow filtration (TFF) with multiple pump passes and a recirculation skid to attain the desired product concentration. The framework process performs a batch UFDF frequently to avoid the need for a large system.  The entire UF/DF process will be split into as many as 20 sub-batches that are expected to be in-sync with the VRF process. A concentration step is operated continuously as the run progresses. The filter is sized to allow for 18 hours per day to give ample time for daily sanitization if required.  Periodically, the concentrated pool is moved to the DF retentate tank where the diafiltration is performed. The material is moved to the final in-process retentate pool where the final UF concentration occurs continuously as the run progresses. Table 1 and 2, depicts the process parameters and throughput for a typical TFF module.  The size of the filters and pumps used is similar across all scales by controlling the frequency of the batch operation. The filter used for the clinical 6 kg DF is 0.2-1 m2 and for the commercial 500kg batch is 0.4-4m2. Each are sub batched one time and twenty times, respectively.

It is apparent from the Table 2 (and SI Table) that the volume of DF buffer required for a batch with high-productivity can be prohibitively high. For the 500kg batch, the DF buffer volume is 66,000L. This issue can be partially mitigated by the use of buffer concentrates and in-line mixing with WFI. Ultimately, a final UF step will be required to attain the 150 g/L target product concentration. Depending on the various scenarios mentioned in Table 2  the requirements for membrane area for the UF and DF buffer would vary. From these calculations (Table 2and SI ), it is apparent that the requirements for DF buffer and the batch operation can be challenging to implement in high productivity scenarios.

Recently, alternative approaches to overcome shortfalls of the typical UF/DF operation have been reported in the literature. The advent of single-pass tangential flow filtration (SPTFF) membrane filtration modules has made it possible to attain high protein concentration factors in a ‘single-pass’ while eliminating the need for repeated product recirculation (Jungbauer, 2013). The use of multistage SPTFF cascade for continuous diafiltration of proteins using cocurrent and countercurrent mode have been demonstrated (Arnold et al., 2019; Huter and Strube, 2019; Jabra et al., 2019; Nambiar et al., 2018; Rucker-Pezzini et al., 2018). Table X2 describes the proposed implementation of the SPTFF modules for an integrated and continuous UF and countercurrent-DF (CC-DF) process. Here, the feed flow to the UF process is adjusted in a way to spread out each VRF sub-batch pool over 24 h. This allows the UF process to be operated uninterrupted over the entire batch. Besides, the relatively low flow also reduces the membrane area required to attain the target product concentration. The retentate stream is then directly fed to the three-stage CC-DF cascade eliminating the need for a surge tank. The DF buffer's countercurrent flow results in at least a 40% reduction in buffer usage than batch DF operation , primarily because the DF step operates at a higher product concentration.

A few pumps are mismatched between the UFDF and SP options. Most pumps in the sub-batch operation match those required for the single-pass operation, between 0.1 and 3 L/min.  The diafiltration crossflow pump requires flow rates up to 26L/min for the 500kg batch. This crossflow pump cannot be used for the single-pass operation. The single-pass operation requires flow rates as low as 33mL/min for the product stream after the initial SPTFF. If the pressure drop through the SPTFF and ILDF modules is sufficiently low, then low flow rate pumps may not be required. Alternatively, the pumps used on the UFDF and SP module could be peristaltic pumps with high-flow and low-flow tubing.

The UFDF/SPTFF skid can use four pumps for both the batch-wise UFDF and the single-pass operation. It would require at least one larger pump for the batch-wise UFDF and possibly one very small pump for the ILDF. Careful design of such a multipurpose skid will allow a facility to migrate between batch or continuous processes flexibly. The integrated UF and DF process lowers the requirements for membrane area and DF buffer regardless of productivity scenarios.

Recent reports also successfully demonstrated the use of hollow-fiber dialyzers for buffer exchange (Yehl et al., 2019). The framework skid would also enable the operation of this step, which has the potential of reducing the total DF buffer required by a factor of 8 from the batch-wise UFDF option.

# Conclusion

The common integrated and continuous bioprocessing framework enables the design of equipment flexible enough to process 6kg to 500kg lot sizes. The design also allows flexibility for some batch operations and is compatible with legacy processes.

It is now possible to design off-the-shelf equipment that would fit most companies ICB. Items and capabilities unique to a company can be upgraded in many cases. Also, it’s possible for contract manufactures to buy equipment that would be backward compatible with batch processes, as well as future compatible for ICB. This flexibility has not been realized to date. Many companies think of ICB processes as a step forward from which there is no turning back. This paper demonstrates that the common framework ICB can be designed, purchased, and installed with an eye toward the future, and yet without worry about the past.

Bakhshayeshi M, Zydney AL. 2008. Effect of solution pH on protein transmission and membrane capacity during virus filtration. Biotechnol. Bioeng. 100:108–117.

Bohonak DM, Zydney AL. 2005. Compaction and permeability effects with virus filtration membranes. J. Memb. Sci. 254:71–79.

Bolton G, Cabatingan M, Rubino M, Lute S, Brorson K, Bailey M. 2005. Normal-flow virus filtration: detection and assessment of the endpoint in bioprocessing. Biotechnol. Appl. Biochem. 42:133.

Ireland T, Bolton G, Noguchi M. 2005. Optimizing Virus Filter Performance  with Prefiltration. Bioprocess Int. 3:44–7.

Rathore AS, Kumar V, Arora A, Lute S, Brorson K, Shukla A. 2014. Mechanistic modeling of viral filtration. J. Memb. Sci. 458:96–103.

Strauss D, Goldstein J, Hongo-Hirasaki T, Yokoyama Y, Hirotomi N, Miyabayashi T, Vacante D. 2017. Characterizing the impact of pressure on virus filtration processes and establishing design spaces to ensure effective parvovirus removal. Biotechnol. Prog. 33:1294–1302.

Syedain ZH, Bohonak DM, Zydney AL. 2006. Protein Fouling of Virus Filtration Membranes: Effects of Membrane Orientation and Operating Conditions. Biotechnol. Prog. 22:1163–1169.

Tateishi J, Kitamoto T, Mohri S, Satoh S, Sato T, Shepherd A, Macnaughton MR. 2001. Scrapie Removal using Planova®Virus Removal Filters. Biologicals 29:17–25.

Wickramasinghe SR, Stump ED, Grzenia DL, Husson SM, Pellegrino J. 2010. Understanding virus filtration membrane performance. J. Memb. Sci. 365:160–169.

References for UF/DF

Arnold L, Lee K, Rucker‐Pezzini J, Lee JH. 2019. Implementation of Fully Integrated Continuous Antibody Processing: Effects on Productivity and COGm. Biotechnol. J. 14:1800061.

Huter MJ, Strube J. 2019. Model-Based Design and Process Optimization of Continuous Single Pass Tangential Flow Filtration Focusing on Continuous Bioprocessing. Processes 7:317.

Jabra MG, Yehl CJ, Zydney AL. 2019. Multistage continuous countercurrent diafiltration for formulation of monoclonal antibodies. Biotechnol. Prog. 35:6–11.

Jungbauer A. 2013. Continuous downstream processing of biopharmaceuticals. Trends Biotechnol. 31:479–492.

Nambiar AMK, Li Y, Zydney AL. 2018. Countercurrent staged diafiltration for formulation of high value proteins. Biotechnol. Bioeng. 115:139–144.

Rucker-Pezzini J, Arnold L, Hill-Byrne K, Sharp T, Avazhanskiy M, Forespring C. 2018. Single Pass Diafiltration Integrated into a Fully Continuous mAb Purification Process. Biotechnol. Bioeng.

Yehl CJ, Jabra MG, Zydney AL. 2019. Hollow fiber countercurrent dialysis for continuous buffer exchange of high-value biotherapeutics. Biotechnol. Prog. 35:1–5.

1

Large scale suspension culture of mammalian cells

Alldread, R. M., J. R. Birch, H. K. Metcalfe, S. Farid, A. J. Racher, R. J. Young,M. J. I. S. S. C. o. L. C. Khan

2014: 410-462.

2

Very high density of Chinese hamster ovary cells in perfusion by alternating tangential flow or tangential flow filtration in WAVE Bioreactor-part II: Applications for antibody production and cryopreservation

Clincke, M. F., C. Molleryd, P. K. Samani, E. Lindskog, E. Faldt, K. Walsh,V. Chotteau

Biotechnol Prog 2013 29(3): 768-777.

https://www.ncbi.nlm.nih.gov/pubmed/23436783

3

Very high density of CHO cells in perfusion by ATF or TFF in WAVE bioreactor. Part I. Effect of the cell density on the process

Clincke, M. F., C. Molleryd, Y. Zhang, E. Lindskog, K. Walsh,V. Chotteau

Biotechnol Prog 2013 29(3): 754-767.

https://www.ncbi.nlm.nih.gov/pubmed/23436789

4

Effects of Cell Density and Temperature on Oxygen Consumption Rate for Different Mammalian Cell Lines.

Jorjani, P.,S. S. Ozturk

Biotechnology and Bioengineering 1999 64(3): 349-356.

5

Jostock, T. (2011). Expression of antibody in mammalian cells. Antibody Expression and Production, Springer: 1-24.

6

Industrialization of mAb production technology: the bioprocessing industry at a crossroads

Kelley, B.

MAbs 2009 1(5): 443-452.

https://www.ncbi.nlm.nih.gov/pubmed/20065641

7

Heat flux and the calorimetric-respirometric ratio as measures of catabolic flux in mammalian cells

Kemp, R. B.,Y. Guan

Thermochimica acta 1997 300: 199-211.

8

Cell culture processes in monoclonal antibody production

Li, F., A. Shen, A. J. P. S. E. D. D. Amanullah, Development,,Manufacturing

2010: 1-38.

9

Cell culture processes for monoclonal antibody production

Li, F., N. Vijayasankaran, A. Y. Shen, R. Kiss,A. Amanullah

MAbs 2010 2(5): 466-479.

https://www.ncbi.nlm.nih.gov/pubmed/20622510

10

EXPERIMENTAL DETERMINATION OF THE VOLUMETRIC MASS TRANSFER COEFFICIENT

Moutafchieva, D., D. Popova, M. Dimitrova,S. Tchaoushev

Journal of Chemical Technology and Metallurgy 2013 48(4): 351-356.

11

Computational fluid dynamic modeling of alternating tangential flow filtration for perfusion cell culture

Radoniqi, F., H. Zhang, C. L. Bardliving, P. Shamlou,J. Coffman

Biotechnol Bioeng 2018 115(11): 2751-2759.

https://www.ncbi.nlm.nih.gov/pubmed/30080936

12

Shear contributions to cell culture performance and product recovery in ATF and TFF perfusion systems

Wang, S., S. Godfrey, J. Ravikrishnan, H. Lin, J. Vogel,J. Coffman

J Biotechnol 2017 246: 52-60.

https://www.ncbi.nlm.nih.gov/pubmed/28159614

13

Wolf, M. K. F., J. M. Bielser,M. Morbidelli (2020). Perfuson Cell Culture Process for Biopharmaceuticals. Cambridge, Cambridge University Press.

14

High Cell Density Perfusion Culture has a Maintained Exoproteome and Metabolome

Zamani, L., M. Lundqvist, Y. Zhang, M. Aberg, F. Edfors, G. Bidkhori, A. Lindahl, A. Mie, A. Mardinoglu, R. Field, R. Turner, J. Rockberg,V. Chotteau

Biotechnol J 2018 13(10): e1800036.

https://www.ncbi.nlm.nih.gov/pubmed/29957873