Abstract

There is a growing interest in using continuous and integrated bioprocessing (ICB) as a manufacturing platform for the production of recombinant protein therapeutics produced from mammalian cells.

At first glance, the newly evolved ICB has created a vast diversity of platforms. A closer inspection reveals convergent evolution: nearly all of the major continuous and integrated bioprocessing methods have a common denominator that would allow manufacturing across a global ecosystem of manufacturers using simple, yet effective, equipment designs. This common denominator is not as efficient or productive as the many sub-species of ICB promoted by those active in this space. The common denominator is, however, capable of supporting the manufacturing of ICB without major changes in the regulator licensure, while achieving most of the efficiency and productivity benefits of the more varied ICB processes.

The common denominator lends itself to implementation in a stepping-stone approach. Manufacturers can implement single-column (SCC) then multicolumn capture (MCC)  capture in stages throughout the lifecycle of a project. The change from SCC to MCC is a relatively minor one for a Protein A capture step that can be introduced in late stage development.

This paper reviews the ICB processes that are being used, or are soon to be used, in a GMP manufacturing setting for recombinant protein production. It describes the design basis, including flow rates, column sizes, material production, buffer and media volumes,  of the common denominator, and discusses the perturbations necessary for fitting various processes into it. It describes the efficiency and productivity of the common denominator process.

- Audience:

  - New Biotech considering Integrated and Continuous Bioprocessing (IBC)

  - CMOs considering how to prepare themselves and what to buy for new IBC clients

  - Vendors of IBC equipment considering designing off-the-shelf equipment for IBC

- Goal: To provide a design basis to create a global IBC ecosystem enabling like-for-like equipment and manufacturing facilities, enabling tech transfer without (significant) changes to CMC sections of an IND or BLA

- Current manufacturing and next gen manufacturing

  - IBC is high productivity, continuous upstream and linked and automated downstream

  - Most use perfusion bioreactor and multicolumn chromatography downstream

- High level NGM framework

  - Review of publicly disclosed IBC (see table and figure)

|  | N-1 | N Brx | # ProA Col. | Virus inact. | #Col. First Polishing | #Col. Second Polishing | Virus Filter | UFDF | GMP? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Amgen | ? | Perf | 1 | Batch | 1 | 1 | Batch | Batch | Y |
| Sanofi | TBD | Perf | ≥2 | Both | 2 | 2 | Cont. | Cont. | Y |
| Novartis | Perf | Perf | 3 | Batch | 1 | 0-1 | Cont. | Cont. | Y |
| BMS | Perf | Batch | ≥2 | Batch | 1 | 1 | Batch | Batch | Y |
| Merck (MSD) | Perf | Perf | ≥2 | Batch | ≥2 | ≥2 | Cont. | Batch | Y |
| Merck (KgaA) | ? | Batch | ≥2 | Cont. | ≥2 | ≥2 | Batch | Batch | Y |
| Pfizer | Perf | Perf | 2 | Cont. | 1 | 0 | Batch | Batch | N |
| Bayer | ? | Batch | ≥2 | Cont. | ≥2 | ≥2 | ? | ? | ? |
| AstraZeneca | Perf | Perf | 2 | TBD | 1 | 0-1 | TBD | TBD | N |

- Bioreactor (one section)

  - Review of bioreactors used in perfusion bioreactors (Ken, Jeong)

    - What cell densities have people reported? At what scale? What is the productivy of a mAb? What VVD have people reported? What is the smallest VVD?

    - Design basis for perfusion bioreactors at 500, 2000L and 12kL scale (Nick)

      - KlA needed vs. cell density? Supplemental data: Plot range representing min/max kLa needed vs.cell density. Plot CO2 stripping needed vs. cell density. What do we know about the normal range of OUR for CHO? .

      - Review range of specific OUR for CHO

      - Review O2 supply methods (microspargers, drilled hole, open pipe, dual sparge strategies)—one or two paragraphs

      - CO2 stripping citations

    - Can we demo this on the NIST mab? Can we do this at NIIMBL?

    - Describe what may need to be added to basic design to accommodate different companies’ platforms.

  - Feed/perfusion media strategy (Jeong) (2-3 paragraphs)

    - Describe how to convert a fed batch into a perfusion media, v1.0

      - Basel + Feed Media strategies

    - Media concentrates v 2.0

  - Bleed strategy and Steady State (1.0) vs. dynamic perfusion (2.0)

  - Review PAT needed (three or four paragraphs)

    - Discuss Titer PAT (HPLC vs. spectroscopy)

    - Glucose/AA feed strategy PAT

    - Discuss Glycosylation (HPLC vs. Spectroscopy)

    - discuss Other PQ

- Harvest skid design (Adrian) (a whole section)

  - Review ATF and TFF

    - Design basis for hollow fiber ATF/TFF (shear, flow rate, typical vvm, typical fluxes, typical membrane capacity) for 500L, 2000L, and 12kL

    - Citations and description of Product retention issue

    - Citation and description of Starling flow

    - Describe what may need to be added to basic design to accommodate different companies’ platforms.

- Step-Wise NGM implementation (short section)

  - Design basis for one column with harvest tank (Jon) (0.5kg/day to 20kg/day)

- Generic Two-column skid design for 500L to 2000L bioreactors, 0.5kg/day to 20kg/day downstream (section)

  - Review MCC methods and benefit to company (Sevda)

    - MCC Capture ProA

    - Counter Current MCC for polishing steps (brief) Morbedelli et al

      - Buffer use per kg of mAb

      - Separation factor needed (alpha = Kp_mab/Kp_impurity, typically >30). As low as 1.1

    - Why two columns? (valve count and pump count)

    - Cost of two column v. one column systems (4x one column systems)

    - Buffer use

  - Design basis for 0.5kg/day to 20kg/day skid

    - Pump sizes

    - Capacity

    - Sensors

    - Describe what may need to be added to basic design to accommodate different companies’ platforms for chromatography

- Continuous virus inactivation skid design (shorter section)

  - Design basis for 0.5kg/day to 20kg/day

  - Detergent inactivation for load to Protein A

  - Acid inactivation for ProA peak

  - Review of Validation strategy: Goal: can we validate virus inactivation in batch and apply it to continuous

  - Review of continuous plug flow reactors [Elias to review] (flow rates are between 100ml/min to 20L/min), (100-500ml/min, 500-2LPM)

    - BI/Pfizer

    - Bayer

    - Sanofi (SEC column)

    - Plug flow reactor design

    - Solid particles about 100um

    - Static mixers

    - Summarize residence time distribution, PFR volume vs. product contact volume, range of flowrates allowable (BI: 50ml/min to 200ml/min; bayer 10-50ml/min)

  - Review characterization of plug flow reactors [Elias to review]

    - Report representative flow rates

    - Report HETP (axial dispersion) HETP (axial dispersion)

- Review of automated batch virus inactivation methods

- Pump sizes for 0.5kg-20kg/day at 10-50g/L product concentration

- Capacity

- Sensors

- SPTFF skid design for UF and DF: review SPTFF and DF SPTFF methods, including patent applications (Sevda) (very short section)

- VRF skid design strategy (daily vs. sub-lots) (very short section)

- Buffer/media prep (two paragraphs)

  - Review buffer dilution

  - Review buffer conditioning

- The future: True Continuous Downstream (NOT SMB)

  - Precipitation based separations

    - Przybyzcien, Zydney, Coffman

    - Alois Jungbauer

  - Chomatan (Zydney)

  - ATPE (Boehringer Ingelheim, …)

Conclusions

- What is the one sentence you’d put in the review article that would summarize the paper?

  - /mr "MCC uses 30% less solution volume than single column chromatography"

- What data from the paper would you cite?

- What are the major conclusions from the paper relevant to CHO cell perfusion, harvest, and integrated chromatography-based downstream?

Nearly any manufacturing facility can take a major step to next generation manufacturing by converting a fed-batch bioreactor to a perfusion bioreactor. For instance, AstraZeneca attached an alternating tangential flow filtration unit on a 500L stainless steel bioreactor in the GMP Gaithersburg Pilot Facility (GPF). In some cases, a larger port may need to be added. Tangential-flow filtration systems can be added to larger bioreactors. The disk-stack centrifuge was bypassed and the existing harvest tank was used as a surge tank for the capture step.

(Andersson, Löfgren et al. 2017) describe an integrated column sequence (ICS), where the product stream is not pooled between steps, but the elution stream from one column is directly loaded onto the next column, often after in-line titration or ionic-strength adjustment, called in-line product conditioning. Since the elution stream from the previous column has variable product concentration and often variable pH and ion strength, conditioning the product stream to a controlled pH and ionic strength can be challenging (Shamashkin, Godavarti et al. 2013). Andersson et al achieved product conditioning at the lab scale by inserting a gel-filtration column to facilitate buffer exchange. This results in a process with N purification columns requiring a total of 2N-1 columns, with associated pumps, valves, and buffers. This system is useful for automating ICB at lab scale, early in the development process or perhaps during linking characterization studies.

The use of desalting columns avoids the apparent complexity of a surge tank between the chromatography steps. The cost is additional buffer volume is approximately 15-20 times the volume of the eluting column1 which approximately doubles the volume of buffer required for this step. The additional automation required is an extra two valves (for the equilibration/load inlet and for the waste/product outlet) and an extra pump. Additionally, the step receiving the product stream must be in a state synced to the gel filtration step to allow loading at the appropriate time.

1Elution volume: 1.5-2CV

Effective desalting volume required: 3.3 CV, so the gel filration column is typically 5-7times the size of the elution column

Equilibration of desalting column: 3CV, so the equilibration solution is typically 15-20 CV

(Arnold, Lee et al. 2019)

Arnold, Lee, et al demonstrated at pilot scale an end-to-end mAb purification consisting of a perfusion bioreactor with bleed-controlled cell density, an ATF harvest system, a five-column Protein A capture, a continuous virus inactivation using a SEC column as a plug-flow reactor, two polishing membrane chromatography steps with no surge tank,  a continuous virus reduction filter, and a single-pass three-stage counter current UFDF system (see Table below).

They modeled a commercial facility with four 12,500L bioreactors capable of 50kg batches, or 3 tons per year, and compared it to same cell line operated in the perfusion mode,  which achieved 50x60kg batches per year. The capital cost for the perfusion-based plant was about 50% of the fed-batch plant. The overall COGM was about 10-15% lower.

5.6g/L titer

70% yield

60 batches per year

2.94 tons per year

Perfusion of 14-42 days, with 1.4g/L (at 30 days) , but no mention of perfusion rate!!! Does mention that the normalized productivity was between 0.79 and 1.07 that of the 12,500L scale.

Batch size for 30 day culture, 50 batches per year= 1.02*2.94=3.00 tons, or 60kg batches.

(Bisschops 2015)

(Cataldo, Burgstaller et al. 2020)

(Coffman, Kramarczyk et al. 2008) Coffman et al describe using high throughput robotic systems for the development of chromatographic steps.

(Dobson, Devine et al. 2016)

(Doshi, Demeule et al. 2015)

(Du, Treiber et al. 2015)

Dynamic perfusion, or non-steady state perfusion, which does not control cell density through bleeding cells from the bioreactor (reference), often uses methods other than bleeding to control the cell density. High osmolarity (ref), pH (ref), decreased temperature (ref), metabolic inhibitors (Pfizer ref), as well as small molecule drug-like inhibitors (Du, Treiber et al. 2015) have been used.

(Dutta, Fedorenko et al. 2017)

(Esmonde-White, Cuellar et al. 2017)

(Fedorenko, Dutta et al. 2020) demonstrated a continuous counter-current Protein A step that resulted in acceptable HCP and DNA removal with good recovery. The productivity with respect to resin volume was significantly higher than standard batch chromatography; the productivity with respect to buffer was much poorer: 2000kg buffer per kg of mAb produced. This Product Mass Index (PMI) for one step is roughly four times more than a standard Protein A step, 570kg/kg. It is roughly the same as the entire three-step platform process of 1,800kg/kg. This level of buffer usage makes this method unusable for next generation manufacturing.

(Fisher, Kamga et al. 2019) reviewed integrated and continuous processing. They concluded that as of the writing of the article, no end-to-end GMP processes had been published.

(Flores, Wang et al. 2019)

(Gjoka, Gantier et al. 2017)

(Godawat, Brower et al. 2012)

(Godawat, Konstantinov et al. 2015)

(Holzberg, Watson et al. 2018)

(Hori and Unno 2019)

(Hummel, Pagkaliwangan et al. 2019)

(Kaltenbrunner 2018)

(Kamga, Cattaneo et al. 2018) Kamga, Cattaneo et al. describe a single-column MabSelect Protein A capture step that is made continuous by the use of a surge tank prior to the column. They use a residence time of one minute, which is about four times faster than typical commercial chromatographic flow rates,  to achieve a full chromatographic cycle in about 35 minutes. While this accelerated flow rate increases the total resin productivity from about 5g/L/hour to 21g/L/hour, it decreases significantly the capacity of the column. Therefore the PMI is about 2000kg/kg product, which is unacceptably high for a single step.

The method has significant value if the flow rate is decreased to normal commercial flow rates, even though the productivity per liter of resin decreases by a factor of four. In addition, instead of using MabSelect with a maximum capacity of about 35mg/mL, PRISMA Protein A can be used with a maximum capacity of 60g/L with a four-minute residence time. This delivers a PMI of 420kg buffer/kg product.

Calc

1cv=1 minute load, at 1.25g/L titer on last 17 days of harvest. Overall average titer looks like less tha 1g/L. 1.25g/L for 50mL onto a 5mL column is about 12.5g/L load.

10CV wash

5CV elution

10CV equilibration

10CV load

25L buffer/Lcolumn total liquid to deliver 12.5g/L column load, or about 2L buffer/g material, or 2000kg/kg PMI.

25L buffer/PRISMA to deliver 60g/L = 420kg/kg PMI

Productivity of resin: 12.5g/L/35min= 21.4g/L/hour

(Karst, Scibona et al. 2017)

(Konstantinov and Cooney 2015)

Konstantinov and Cooney describe “A unit operation is continuous if it is capable of processing a continuous flow input for prolonged periods of time. A con- tinuous unit operation has minimal internal hold volume. The output can be continuous or discretized in small packets pro- duced in a cyclic manner.

A process is continuous if it is composed of integrated (physically connected) continuous unit operations with zero or minimal hold volume in between. To emphasize that all the unit operations are continuous and integrated, such processes are also referred to as fully continuous or end-to-end continuous. A process is hybrid if it is composed of both batch and continuous unit operations.”

“Continuous processing offers significant opportunity to reduce capital costs through radical reduction of facility and equipment foot- prints that result from: small bioreactors and chromatography columns; elimination of intermediate hold tanks and nonvalue- added unit operations; very high-volumetric productivity; high equipment utilization rate; and single-use technology. The vol- ume of the continuous bioreactors and purification columns can be multifold smaller than the corresponding fed-batch equip- ment because of significantly higher cell density in perfusion systems and the downstream equipment utilization rate.”

PAT:

Different companies will have different PAT needs depending upon their platform. All need the standard PAT methods we use today plus others. Companies that do perfusion will need hourly(or so) measurements of cell density. Many companies will have a feed-back loop to control cell density based on this measurement. Many will also feed based on this measurement. Catabolite Measurement: like glucose or AA Titer measurement: for feed-forward control of the load to the capture step My dream PAT: hourly feedback of glyco profile to allow feedback control Downstream options we're looking into most actively: Fast (~1 s) pH measure with NO CALIBRATION FOR 20 DAYS. for Virus inactivation. This does not exist to my knowledge Measures of HMW for feed forward to CEX every ten minutes or so; UPLC can provide this just fine Measure HMW for peak shaving every 10 seconds or so. Light scattering is probably fine here; we have no pilot experience with this in-line Measure of HCP for feed-forward control of AEX step every 10 minutes or so. This does not exist yet.

(Kumar, Gammell et al. 2007)

(Löfgren, Andersson et al. 2018)

(Löfgren, Yamanee-Nolin et al. 2019)

(Lute, Kozaili et al. 2020)

(Mayrhofer and Kunert 2020)

(Pinto, Napoli et al. 2020)

(Pollock, Coffman et al. 2017)

(Rathore, Agarwal et al. 2015)

(Rayat, Chatel et al. 2016)

(Shamashkin, Godavarti et al. 2013)

(Singh and Herzer 2018)

(Tan and Franzreb 2020)

(Tripathi and Shrivastava 2019)

(Walther, Godawat et al. 2015)

(Walther, McLarty et al. 2019)

(Warikoo, Godawat et al. 2012)

(Zelger, Pan et al. 2016)

(Zydney 2015)

(Zydney 2016)

1

Design and control of integrated chromatography column sequences

Andersson, N., A. Löfgren, M. Olofsson, A. Sellberg, B. Nilsson,P. Tiainen

Biotechnology Progress 2017 33(4): 923-930.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85011684970&doi=10.1002%2fbtpr.2434&partnerID=40&md5=cad8249ab4475a4156dfb9764d02e0fc

https://aiche.onlinelibrary.wiley.com/doi/abs/10.1002/btpr.2434

2

Implementation of Fully Integrated Continuous Antibody Processing: Effects on Productivity and COGm

Arnold, L., K. Lee, J. Rucker-Pezzini,J. H. Lee

Biotechnology Journal 2019 14(2).

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85047497641&doi=10.1002%2fbiot.201800061&partnerID=40&md5=0ac4b88f9a22ca76521efff20736e00d

https://onlinelibrary.wiley.com/doi/abs/10.1002/biot.201800061

3

Manufacturing solutions

Backx, I.

European Pharmaceutical Review 2015 20(2): 69.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84929895284&partnerID=40&md5=2ab09cd610b80d958cbc481a73a46995

4

Bisschops, M. (2015). BioSMB Technology as an Enabler for a Fully Continuous Disposable Biomanufacturing Platform. Continuous Processing in Pharmaceutical Manufacturing: 35-52.

5

Economics and ecology: Modelling of continuous primary recovery and capture scenarios for recombinant antibody production

Cataldo, A. L., D. Burgstaller, G. Hribar, A. Jungbauer,P. Satzer

J Biotechnol 2020 308: 87-95.

https://www.ncbi.nlm.nih.gov/pubmed/31809781

6

High-throughput screening of chromatographic separations: I. Method development and column modeling

Coffman, J. L., J. F. Kramarczyk,B. D. Kelley

Biotechnol Bioeng 2008 100(4): 605-618.

https://www.ncbi.nlm.nih.gov/pubmed/18496874

7

Engineering the surface properties of a human monoclonal antibody prevents self-association and rapid clearance in vivo

Dobson, C. L., P. W. Devine, J. J. Phillips, D. R. Higazi, C. Lloyd, B. Popovic, J. Arnold, A. Buchanan, A. Lewis, J. Goodman, C. F. van der Walle, P. Thornton, L. Vinall, D. Lowne, A. Aagaard, L. L. Olsson, A. Ridderstad Wollberg, F. Welsh, T. K. Karamanos, C. L. Pashley, M. G. Iadanza, N. A. Ranson, A. E. Ashcroft, A. D. Kippen, T. J. Vaughan, S. E. Radford,D. C. Lowe

Sci Rep 2016 6: 38644.

https://www.ncbi.nlm.nih.gov/pubmed/27995962

8

Understanding Particle Formation: Solubility of Free Fatty Acids as Polysorbate 20 Degradation Byproducts in Therapeutic Monoclonal Antibody Formulations

Doshi, N., B. Demeule,S. Yadav

Mol Pharm 2015 12(11): 3792-3804.

https://www.ncbi.nlm.nih.gov/pubmed/26419285

9

Use of a small molecule cell cycle inhibitor to control cell growth and improve specific productivity and product quality of recombinant proteins in CHO cell cultures

Du, Z., D. Treiber, J. D. McCarter, D. Fomina-Yadlin, R. A. Saleem, R. E. McCoy, Y. Zhang, T. Tharmalingam, M. Leith, B. D. Follstad, B. Dell, B. Grisim, C. Zupke, C. Heath, A. E. Morris,P. Reddy

Biotechnol Bioeng 2015 112(1): 141-155.

https://www.ncbi.nlm.nih.gov/pubmed/25042542

10

Continuous countercurrent tangential chromatography for mixed mode post-capture operations in monoclonal antibody purification

Dutta, A. K., D. Fedorenko, J. Tan, J. A. Costanzo, D. S. Kahn, A. L. Zydney,O. Shinkazh

Journal of Chromatography A 2017 1511: 37-44.

11

Raman spectroscopy as a process analytical technology for pharmaceutical manufacturing and bioprocessing

Esmonde-White, K. A., M. Cuellar, C. Uerpmann, B. Lenain,I. R. Lewis

Analytical and Bioanalytical Chemistry 2017 409(3): 637-649.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84982918737&doi=10.1007%2fs00216-016-9824-1&partnerID=40&md5=5ece974c688a200860c4bf4d6b4d3e8b

https://www.ncbi.nlm.nih.gov/pmc/articles/PMC5233728/pdf/216_2016_Article_9824.pdf

12

Improved protein A resin for antibody capture in a continuous countercurrent tangential chromatography system

Fedorenko, D., A. K. Dutta, J. Tan, J. Walko, M. Brower, N. D. S. Pinto, A. L. Zydney,O. Shinkazh

Biotechnol Bioeng 2020 117(3): 646-653.

https://www.ncbi.nlm.nih.gov/pubmed/31784975

13

The Current Scientific and Regulatory Landscape in Advancing Integrated Continuous Biopharmaceutical Manufacturing

Fisher, A. C., M. H. Kamga, C. Agarabi, K. Brorson, S. L. Lee,S. Yoon

Trends in Biotechnology 2019 37(3): 253-267.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85053713207&doi=10.1016%2fj.tibtech.2018.08.008&partnerID=40&md5=c2bd30b2ecb1bdb5f1c8f64dddff7017

14

Recent trends in integrated bioprocesses: aiding and expanding microbial biofuel/biochemical production

Flores, A., X. Wang,D. R. Nielsen

Current Opinion in Biotechnology 2019 57: 82-87.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85062806021&doi=10.1016%2fj.copbio.2019.02.007&partnerID=40&md5=11e89497ed3a121d1e3bd106f2ca1dab

15

Platform for integrated continuous bioprocessing

Gjoka, X., R. Gantier,M. Schofield

BioPharm International 2017 30(7): 26-32.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85024104809&partnerID=40&md5=53ae75ad91a92c2c93f1aadb9013c7fa

16

Periodic counter-current chromatography - design and operational considerations for integrated and continuous purification of proteins

Godawat, R., K. Brower, S. Jain, K. Konstantinov, F. Riske,V. Warikoo

Biotechnology Journal 2012 7(12): 1496-1508.

https://onlinelibrary.wiley.com/doi/abs/10.1002/biot.201200068

17

End-to-end integrated fully continuous production of recombinant monoclonal antibodies

Godawat, R., K. Konstantinov, M. Rohani,V. Warikoo

Journal of Biotechnology 2015 213: 13-19.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84942838566&doi=10.1016%2fj.jbiotec.2015.06.393&partnerID=40&md5=e630756a8177806690a32edc9ec163cb

18

Sensors for biomanufacturing process development: facilitating the shift from batch to continuous manufacturing

Holzberg, T. R., V. Watson, S. Brown, A. Andar, X. Ge, Y. Kostov, L. Tolosa,G. Rao

Current Opinion in Chemical Engineering 2018 22: 115-127.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85055201973&doi=10.1016%2fj.coche.2018.09.008&partnerID=40&md5=1dbfef8b700f6a04bfc55b0877664d14

19

Hori, K.,H. Unno (2019). Integrated production and separation. Comprehensive Biotechnology: 651-662.

20

Modeling the Downstream Processing of Monoclonal Antibodies Reveals Cost Advantages for Continuous Methods for a Broad Range of Manufacturing Scales

Hummel, J., M. Pagkaliwangan, X. Gjoka, T. Davidovits, R. Stock, T. Ransohoff, R. Gantier,M. Schofield

Biotechnology Journal 2019 14(2).

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85041724058&doi=10.1002%2fbiot.201700665&partnerID=40&md5=dd3fa89d4845294abf39ce2e59462686

https://onlinelibrary.wiley.com/doi/full/10.1002/biot.201700665

21

Kaltenbrunner, O. (2018). Points to Consider for Design and Control of Continuous Bioprocessing. Biopharmaceutical Processing: Development, Design, and Implementation of Manufacturing Processes: 741-754.

22

Integrated continuous biomanufacturing platform with ATF perfusion and one column chromatography operation for optimum resin utilization and productivity

Kamga, M. H., M. Cattaneo,S. Yoon

Prep Biochem Biotechnol 2018 48(5): 383-390.

https://www.ncbi.nlm.nih.gov/pubmed/29509101

23

Modulation and modeling of monoclonal antibody N-linked glycosylation in mammalian cell perfusion reactors

Karst, D. J., E. Scibona, E. Serra, J. M. Bielser, J. Souquet, M. Stettler, H. Broly, M. Soos, M. Morbidelli,T. K. Villiger

Biotechnol Bioeng 2017 114(9): 1978-1990.

https://www.ncbi.nlm.nih.gov/pubmed/28409838

24

White Paper on Continuous Bioprocessing May 20–21 2014 Continuous Manufacturing Symposium

Konstantinov, K. B.,C. L. Cooney

Journal of Pharmaceutical Sciences 2015 104(3): 813-820.

25

Proliferation control strategies to improve productivity and survival during CHO based production culture : A summary of recent methods employed and the effects of proliferation control in product secreting CHO cell lines

Kumar, N., P. Gammell,M. Clynes

Cytotechnology 2007 53(1-3): 33-46.

https://www.ncbi.nlm.nih.gov/pubmed/19003188

26

Development of small-scale models to understand the impact of continuous downstream bioprocessing on integrated virus filtration

Lute, S., J. Kozaili, S. Johnson, K. Kobayashi,D. Strauss

Biotechnology Progress 2020.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85078833970&doi=10.1002%2fbtpr.2962&partnerID=40&md5=bbf0d185f3a257b6971e3d892e911a68

https://aiche.onlinelibrary.wiley.com/doi/full/10.1002/btpr.2962

27

Designing an Autonomous Integrated Downstream Sequence From a Batch Separation Process − An Industrial Case Study

Löfgren, A., N. Andersson, A. Sellberg, B. Nilsson, M. Löfgren,S. Wood

Biotechnology Journal 2018 13(4).

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85041107635&doi=10.1002%2fbiot.201700691&partnerID=40&md5=8dcb875f1aec8a11dc31431f71ad0de4

https://onlinelibrary.wiley.com/doi/abs/10.1002/biot.201700691

28

Optimization of integrated chromatography sequences for purification of biopharmaceuticals

Löfgren, A., M. Yamanee-Nolin, S. Tallvod, J. G. Fons, N. Andersson,B. Nilsson

Biotechnology Progress 2019 35(6).

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85068480067&doi=10.1002%2fbtpr.2871&partnerID=40&md5=30e070acf2a3f6214455afadc224adb5

https://aiche.onlinelibrary.wiley.com/doi/full/10.1002/btpr.2871

29

Mayrhofer, P.,R. Kunert

Methods in Molecular Biology 2020 2095: 27-39.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85077164304&doi=10.1007%2f978-1-0716-0191-4_3&partnerID=40&md5=43d98f3be178ff7fc40bdeafa6a2eec9

https://link.springer.com/protocol/10.1007%2F978-1-0716-0191-4_3

30

Impact of micro and macroporous TFF membranes on product sieving and chromatography loading for perfusion cell culture

Pinto, N. D. S., W. N. Napoli,M. Brower

Biotechnology and Bioengineering 2020 117(1): 117-124.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85075203427&doi=10.1002%2fbit.27192&partnerID=40&md5=4cbf3c1a6b5a613f1ee2d328762ca767

https://onlinelibrary.wiley.com/doi/full/10.1002/bit.27192

31

Integrated continuous bioprocessing: Economic, operational, and environmental feasibility for clinical and commercial antibody manufacture

Pollock, J., J. Coffman, S. V. Ho,S. S. Farid

Biotechnology Progress 2017 33(4): 854-866.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85020083934&doi=10.1002%2fbtpr.2492&partnerID=40&md5=65d90b6d22e85635a47c1ccb69e8f952

https://www.ncbi.nlm.nih.gov/pmc/articles/PMC5575510/pdf/BTPR-33-854.pdf

32

Continuous Processing for Production of Biopharmaceuticals

Rathore, A. S., H. Agarwal, A. K. Sharma, M. Pathak,S. Muthukumar

Preparative Biochemistry and Biotechnology 2015 45(8): 836-849.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84941071281&doi=10.1080%2f10826068.2014.985834&partnerID=40&md5=cae0a37c7f0e9a02901adc0a52b5ebdf

https://www.tandfonline.com/doi/full/10.1080/10826068.2014.985834

33

Ultra scale-down approaches to enhance the creation of bioprocesses at scale: impacts of process shear stress and early recovery stages

Rayat, A. C., A. Chatel, M. Hoare,G. J. Lye

Current Opinion in Chemical Engineering 2016 14: 150-157.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84993977917&doi=10.1016%2fj.coche.2016.09.012&partnerID=40&md5=c443eeaadcf0538fa15d797d175c6505

34

A tandem laboratory scale protein purification process using Protein A affinity and anion exchange chromatography operated in a weak partitioning mode

Shamashkin, M., R. Godavarti, T. Iskra,J. Coffman

Biotechnology and Bioengineering 2013 110(10): 2655-2663.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84896406212&doi=10.1002%2fbit.24955&partnerID=40&md5=0135bc36472025af3d776a2d29f8659c

https://onlinelibrary.wiley.com/doi/full/10.1002/bit.24955

35

Singh, N.,S. Herzer

Advances in Biochemical Engineering/Biotechnology 2018 165: 115-178.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85059032532&doi=10.1007%2f10_2017_12&partnerID=40&md5=452516fa87ceb171fc622fcf5e0fcf0d

https://link.springer.com/content/pdf/10.1007%2F10_2017_12.pdf

36

Continuous ultrafiltration/diafiltration using a 3D-printed two membrane single pass module

Tan, R.,M. Franzreb

Biotechnol Bioeng 2020 117(3): 654-661.

https://www.ncbi.nlm.nih.gov/pubmed/31788780

37

Recent Developments in Bioprocessing of Recombinant Proteins: Expression Hosts and Process Development

Tripathi, N. K.,A. Shrivastava

Frontiers in Bioengineering and Biotechnology 2019 7.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-85077373424&doi=10.3389%2ffbioe.2019.00420&partnerID=40&md5=084d76f1353e354d77f652d9ab7ca6e7

https://fjfsdata01prod.blob.core.windows.net/articles/files/496566/pubmed-zip/.versions/1/.package-entries/fbioe-07-00420/fbioe-07-00420.pdf?sv=2015-12-11&sr=b&sig=WfKTfc42NGf8fI9FuyTGytDO6KOG%2BHpjgYck%2BHSYukI%3D&se=2020-04-10T20%3A29%3A38Z&sp=r&rscd=attachment%3B%20filename%2A%3DUTF-8%27%27fbioe-07-00420.pdf

38

The business impact of an integrated continuous biomanufacturing platform for recombinant protein production

Walther, J., R. Godawat, C. Hwang, Y. Abe, A. Sinclair,K. Konstantinov

Journal of Biotechnology 2015 213: 3-12.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84942824422&doi=10.1016%2fj.jbiotec.2015.05.010&partnerID=40&md5=c17d5a50a9e51a5d500cd3b869aa0c8c

39

The effects of alternating tangential flow (ATF) residence time, hydrodynamic stress, and filtration flux on high-density perfusion cell culture

Walther, J., J. McLarty,T. Johnson

Biotechnol Bioeng 2019 116(2): 320-332.

https://www.ncbi.nlm.nih.gov/pubmed/30080929

40

Integrated continuous production of recombinant therapeutic proteins

Warikoo, V., R. Godawat, K. Brower, S. Jain, D. Cummings, E. Simons, T. Johnson, J. Walther, M. Yu, B. Wright, J. McLarty, K. P. Karey, C. Hwang, W. Zhou, F. Riske,K. Konstantinov

Biotechnology and Bioengineering 2012 109(12): 3018-3029.

https://onlinelibrary.wiley.com/doi/full/10.1002/bit.24584

41

Real-time monitoring of protein precipitation in a tubular reactor for continuous bioprocessing

Zelger, M., S. Pan, A. Jungbauer,R. Hahn

Process Biochemistry 2016 51(10): 1610-1621.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84977626973&doi=10.1016%2fj.procbio.2016.06.018&partnerID=40&md5=4ed195edb66d4606ea6a101604968831

42

Perspectives on integrated continuous bioprocessing - opportunities and challenges

Zydney, A. L.

Current Opinion in Chemical Engineering 2015 10: 8-13.

https://www.scopus.com/inward/record.uri?eid=2-s2.0-84937888574&doi=10.1016%2fj.coche.2015.07.005&partnerID=40&md5=b49c55a3d83cfd63b42e0373af070b94

43

Continuous downstream processing for high value biological products: A Review

Zydney, A. L.

Biotechnology and Bioengineering 2016 113(3): 465-475.

|  | Bioreactor | Cell density control | Harvest | Capture Column | Virus Inactivation | Surge Tank | Polishing 1 | Surge Tank | Polishing 2 | VRF | Surge Tanks | UFDF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arnold, Lee et al 2019 | Perfusion | Bleed | ATF | 5-column Protein A | Continuous (SEC column plug flow reactor) | Yes, and SPTFF | 1 AEX membrane | No | 1 CEX membrane | In-line, continuous | Yes | CC-SPTFF |

COGM and CAPEX

| Citation | Batch scale | ICB scale | Mass/year | Capital cost | COGM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Arnold, Lee et al 2019 | 4x12,500 | 5x2000L | 3 tons | 50% lower | 11-15% lower |  |  |