Abstract: There is a growing interest in using continuous and integrated bioprocessing (ICB) as a manufacturing platform for the production of recombinant protein therapeutics produced from mammalian cells. The current platform for monoclonal antibodies, bispecific antibodies, and some Fc-fusion proteins is based on fed-batch cell culture. The platform is consistent enough from company to company to allow the transfer of processes across a global ecosystem of manufacturers. Off-the-shelf equipment is available from many sources that allow the turn-key implementation of batch processing.

At first glance, the relatively newly evolved ICB has created a vast diversity of incompatible platforms. A closer inspection reveals convergent evolution: nearly all of the major continuous and integrated bioprocessing methods have a common denominator that would allow manufacturing across a global ecosystem of manufacturers using simple, yet effective, equipment designs. This common denominator is not quite as efficient or productive as the many sub-species of ICB promoted by those active in this space. The common denominator is, however, capable of supporting the manufacturing of ICB without major changes in the BLA, while achieving most of the efficiency and productivity benefits of the more varied ICB processes.

This paper reviews the ICB processes that are being used, or are soon to be used, in a GMP manufacturing setting for recombinant protein production. It describes the design basis of the common denominator, and discusses the perturbations necessary for fitting various processes into it. It describes the efficiency and productivity of the common denominator process.

Abstract:

  - Audience:

    - New Biotech considering NGM

    - CMOs considering how to prepare themselves and what to buy for new NGM clients

    - Vendors of NGM equipment considering designing off-the-shelf equipment for NMG

  - Goal: To provide a design basis to create a global NGM ecosystem enabling like-for-like equipment and manufacturing facilities, enabling tech transfer without (significant) changes to CMC sections of an IND or BLA

  - Current manufacturing and next gen manufacturing

    - NGM is high productivity linked and connected processing

    - Most use perfusion bioreactor and multicolumn chromatography downstream

  - High level NGM framework

    - Review of publicly disclosed NGM (see table and figure)

  - Bioreactor

    - Review of bioreactors used in perfusion bioreactors

      - Not extensively reviewed: low cell density, generation 1 perfusion

      - What cell densities have people reported? At what scale? What is the productivy of a mAb? What VVD have people reported? What is the smallest VVD?

      - Design basis for perfusion bioreactors at 500, 2000L and 12kL scale (Nick)

        - KlA needed vs. cell density? Plot range representing min/max kLa needed vs.cell density. Plot CO2 stripping needed vs. cell density. What do we know about the normal range of OUR for CHO? .

        - Review range of specific OUR for CHO

        - Review O2 supply methods

        - Review CO2 stripping literature

      - Can we demo this on the NIST mab?

    - Feed/perfusion media strategy

      - Describe how to convert a fed batch into a perfusion media, v1.0

        - Basel + Feed Media strategies

      - Media concentrates v 2.0

    - Bleed strategy and Steady State (1.0) vs. dynamic perfusion (2.0)

    - Review PAT needed

      - Review Titer PAT (HPLC vs. spectroscopy)

      - Glucose/AA feed strategy PAT

      - Review Glycosylation (HPLC vs. Spectroscopy)

      - Review raw materials

      - Review Other PQ

  - Harvest skid design (Adrian)

    - Review ATF and TFF

      - Design basis for hollow fiber ATF/TFF (shear, flow rate, typical vvm, typical fluxes, typical membrane capacity) for 500L, 2000L, and 12kL

      - Product retention

      - Starling flow

  - Step-Wise NGM implementation

    - Design basis for one column with harvest tank (basically the 6389 implementation--Jon)

      - Can we demonstrate this on the NIST mab?

    - Valve and pump design to convert a single column skid into a two column system

  - Generic Two-column skid design for 500L to 2000L bioreactors, 0.5kg/day to 20kg/day downstream

    - Review MCC methods and benefit to company (Sevda)

      - MCC Capture ProA

      - Counter Current MCC for polishing steps (brief) Morbedelli et al

        - Buffer use per kg of mAb

        - Separation factor needed (alpha = Kp_mab/Kp_impurity, typically >30). As low as 1.1

      - Why two columns? (valve count and pump count)

    - Design basis for 0.5kg/day to 20kg/day skid

      - Pump sizes

      - Capacity

      - Sensors

  - Continuous virus inactivation skid design

    - Design basis for 0.5kg/day to 20kg/day

    - Review of Validation strategy: Goal: can we validate virus inactivation in batch and appy it to continuous

    - Review of continuous plug flow reactors [Elias to review] (flow rates are between 100ml/min to 20L/min), (100-500ml/min, 500-2LPM)

      - BI/Pfizer

      - Bayer

      - Sanofi (SEC column)

      - Liquid liquid extraction packing?

      - Plug flow reactor design

      - Solid particles about 100um

      - Static mixers

      - Summarize residence time distribution, PFR volume vs. product contact volume, range of flowrates allowable (BI: 50ml/min to 200ml/min; bayer 10-50ml/min)

    - Review characterization of plug flow reactors [Elias to review]

      - Report representative flow rates

      - Report HETP (axial dispersion) HETP (axial dispersion)

  - Review of automated batch virus inactivation methods

  - Pump sizes for 0.5kg-20kg/day at 10-50g/L product concentration

  - Capacity

  - Sensors

  - Seed-train design strategies

    - 100 and 500L ATF systems

  - SPTFF skid design for UF and DF: review SPTFF and DF SPTFF methods, including patent applications (Sevda)

  - VRF skid design strategy: daily x`x`

  - Bioburden control strategy

    - Waste trap requirements

    - Surge Tank requirements

    - Sampling plan

    - Slip-stream samples

  - Buffer/media prep

    - Review buffer dilution

    - Review buffer conditioning

  - Downstream Control Strategy

    - Modeling and Machine Learning

    - PAT needed

      - HCP ideally (can show if HCP is removed in many cases, DNA, rProA, virus is removed)

      - HMW

  - Media development from fed batch

  - True Continuous Downstream (NOT SMB)

    - Precipitation based separations

      - Przybyzcien, Zydney, Coffman

      - Alois Jungbauer

    - Chomatan (Zydney)

    - ATPE (Boehringer Ingelheim, …)

  - What is the one sentence you’d put in the review article that would summarize the paper?

    - /mr "MCC uses 30% less solution volume than single column chromatography"

  - What data from the paper would you cite?

  - What are the major conclusions from the paper relevant to CHO cell perfusion, harvest, and integrated chromatography-based downstream?