# A Framework for Integrated and Continuous Processing

By Jonathan Coffman

Abstract

There is a growing interest in using continuous and integrated bioprocessing (ICB) as a manufacturing platform for the production of recombinant protein therapeutics produced from mammalian cells.

At first glance, the newly evolved ICB has created a vast diversity of platforms. A closer inspection reveals convergent evolution: nearly all of the major continuous and integrated bioprocessing methods have a framework built from a common denominator that could allow manufacturing across a global ecosystem of manufacturers using simple, yet effective, equipment designs. This framework is not as efficient or productive as the many sub-species of ICB promoted by those active in this space. The framework is, however, capable of supporting the manufacturing of ICB without major changes in the regulator licensure, while achieving most of the efficiency and productivity benefits of the more varied ICB processes.

The framework lends itself to implementation in a stepping-stone approach. Manufacturers can implement single-column (SCC) then multicolumn capture (MCC)  capture in stages throughout the lifecycle of a project. The change from SCC to MCC is a relatively minor one for a Protein A capture step that can be introduced in late stage development.

This paper reviews the ICB processes that are being used, or are soon to be used, in a GMP manufacturing setting for recombinant protein production. It describes the design basis, including flow rates, column sizes, material production, buffer and media volumes, of the framework, and discusses the perturbations necessary for fitting various processes into it. It describes the efficiency and productivity of the framework process.

# Assumptions

The assumptions in the framework process are described below. These assumptions are typical throughout the industry and do not represent confidential information.

| Bioreactor |  |
| --- | --- |
| Bioreactor Size (L) | 500-2000 |
| Avg cell density (million cells/mL) | 50-150 |
| Qp (pg/cell/day) | 20-60 |
| Permeate Perfusion Rate (bioreactor volumes/day, vvd) | 1.5 |
| Batch Duration | 14-30 |
| Media Concentration Factor | 1-5x |

| General Column Data |  |
| --- | --- |
| Linear flowrate (cm/hr) | 300 |
| Bed Height (cm) | 20 |
| Buffer Concentration Factor | 5 |
| # Columns | 1-2 |

| Chromatography | Protein A | Flow Through | Bind/Elute |
| --- | --- | --- | --- |
| Capacity (g/L) | 30-60 | 50-150 | 30-60 |
| Buffer Volume (1x) used (Column Volume, CV) | 22 | 16 | 16 |
| Elution Volume collected (CV) | 1.5-3 | 0.2-0.5 | 1.0-1.5 |
| Yield | 90% | 95% | 85% |

| Virus Filtration |  |
| --- | --- |
| Capacity (L/m2) | 700-1400 |
| Flux (LMH) | 100 |
| Yield | 95% |

| UFDF |  |
| --- | --- |
| Membrane Capacity (g/m2) | 800-1500 |
| Yield | 95% |
| Cross Flow (LMH) | 300-500 |
| Concentration 1 Flux (LMH) | 30-60 |
| Concentration 1 target (g/L) | 50 |
| # Diavolumes | 6 |
| Final DS Conc Target (g/L) | 100 |