# True Continuous Downstream

## Precipitation Based Separations

The significant increase in product titer that is achieved in perfusion bioreactor demands an alternative method to a chromatographic process, whose performance is reduced at high titers. Precipitation is one of the earliest developed techniques that can be used as an alternative method because of its low cost and higher selectivity as the product titer increases.

Tscheliessnig et al. (2014) used the cold ethanol precipitation (CEP) method for purification of monoclonal antibodies (mAbs) from Chinese hamster ovary (CHO) cell culture supernatant where calcium chloride (CaCl2) was also added to overcome the co-purification of DNA during the CEP process. The developed process was introduced as a continuous operation in which methanol and isopropanol could also be replaced with ethanol in order to reduce the final cost of the process. The experiments were conducted with antibodies of different isoelectric points; however, the level of host cell proteins (HCPs) clearance was different in each case and it varied from 20-90x. Depending on the initial concentration of the antibody that was used, the final yield for the developed process also varied from 60-90% (Tscheliessnig, Satzer et al. 2014).

Li et al. (2019) developed a continuous precipitation process for purification of antibody product from harvested cell culture fluid using a combination of zinc chloride (ZnCl2) as a cross-linking agent, and polyethylene glycol (PEG) as a volume exclusion agent. Precipitation was accomplished in a tubular reactor fitted with a static mixer where ZnCl2 and PEG were added sequentially to provide a uniform precipitate yields and morphologies. To further reduce the amount of soluble impurities, the precipitates were continuously washed using a two-stage counter-current hollow fiber membrane modules that resulted in more than 90% removal of HCPs. During the washing step, HEPES buffer was used not only to reduce the costs but also to eliminate the additional step for removal of ZnCl2 and PEG; however, it slightly decreased the final yield of the process. The precipitated product after washing step was resolubilized using a depth filter by decreasing the pH. During this step, the cross-linking associated with cations was disrupted and the product was directly prepared for a continuous virus inactivation step. The results from this study demonstrated the feasibility of the fully continuous integrated precipitation process with a noticeably short residence time (<30 s), and without any intervening hold steps during precipitation, washing, and resolubilizing (Li, Gu et al. 2019).

Burgstaller et al. (2019) developed a continuous precipitation process consisting of a tubular reactor integrated with two stages of tangential flow filtration (TFF) units, which resulted in 97% antibody purity and a 95% process yield. The cell culture broth, PEG, and ZnCl2 were continuously fed into a tubular reactor setup fitted with a static mixer; note that the total hold time of the precipitation step was 22.5 minutes. Next, the precipitate from the reactor was concentrated continuously by the first stage of the TFF units, and the second stage of the TFF units was used as the following washing step. A 0.2-µm hollow fiber module was used during the two-stage TFF process which produced an overall concentration factor of 8.2 for the product. The described process was introduced as a truly continuous capture step without a need for a surge tank or an in-line protein concentration monitoring (Burgstaller, Jungbauer et al. 2019).

## Continuous Countercurrent Tangential Chromatography

Continuous countercurrent tangential chromatography (CCTC) is considered as a cost saving alternative to a column-based chromatographic processes. Shinkazh et al. (2011) developed the CCTC process where the chromatographic resin was utilized in the form of a slurry and it flowed sequentially through a series of static mixers and microporous hollow fiber membrane modules. All of the dissolved species and impurities passed through the membrane during the binding step, while the target product was retained by binding onto the large resin particle. The binding step was followed by a washing step in order to remove the trace amount of impurities that were retained, and the target product was recovered from the slurry during the final elution step. A multistage countercurrent configuration was used during binding, washing, and elution steps in order to reduce the amount of required buffer (Shinkazh, Kanani et al. 2011).

In order to employ the CCTC technique for capturing the monoclonal antibody from the clarified cell culture harvest, Fedorenko et al. (2020) used a customized small-sized agarose resin (20–25 μm) with lower cross-linking. The results from this study showed a high productivity up to 140 g of mAb.L resin-1.hr-1 due to the increased binding capacity and a faster kinetics comparing with the previous studies, where a commercial chromatography resins were used. A higher buffer usage compared with the batch column operation is considered as one of the main challenges associated with CCTC, where the employed small-sized resins indicated a 2X reduction in buffer consumption for the overall process (Fedorenko, Dutta et al. 2020).

## Aqueous Two-Phase Extraction

Aqueous two-phase extraction (ATPE) is considered as a low-cost technique for a continuous downstream processing, which offers a moderate selectivity and high yield for the purification of biomolecules. One of the early developments of the continuous ATPE, in a pilot scale, was developed by Boland et al. (1990) for purification of bovine liver, using PEG and phosphate systems. The experiments were conducted using two-disc separators, which yielded a 4-fold purification factor with respect to soluble proteins, meanwhile a recovery of 83% was achieved (Boland, Hesselink et al. 1991).

For the purification of antibody from a CHO cells supernatant, Rosa et al. (2012) used a packed-column contactor during the continuous counter-current ATPE process. The initial experiments confirmed the capability of the system to recover 65% of the antibodies and removal of more than 80% of impurities. In order to improve the recovery and purity, a mixer settler was used in this system where the recovery and the protein removal both increased for about 85% (Rosa, Azevedo et al. 2012).

Eggersgluess et al. (2014) developed a continuous countercurrent multi-stage ATPE process to concentrate and purify the antibody from clarified cell culture fluid. During all of the extraction steps 12% PEG and 18% phosphate was used, and the experiments were conducted using a mixer settler in combination with an extraction column. During the initial extraction step, the product volume was reduced up to 5-fold; meanwhile, the purity was less than 40%. In order to improve the purity, a multi-stage wash-extraction step was implemented which resulted in achieving the purity level of 70% and the yield of 95% (Eggersgluess, Richter et al. 2014).

References:

Boland, M. J., et al. (1991). "Extractive purification of enzymes from animal tissue using aqueous two phase systems: pilot scale studies."  19(1): 19-33.

Burgstaller, D., et al. (2019). "Continuous integrated antibody precipitation with two‐stage tangential flow microfiltration enables constant mass flow."  116(5): 1053-1065.

Eggersgluess, J. K., et al. (2014). "Multi‐stage aqueous two‐phase extraction for the purification of monoclonal antibodies."  37(4): 675-682.

Fedorenko, D., et al. (2020). "Improved protein A resin for antibody capture in a continuous countercurrent tangential chromatography system."  117(3): 646-653.

Li, Z., et al. (2019). "Continuous precipitation for monoclonal antibody capture using countercurrent washing by microfiltration." e2886.

Rosa, P., et al. (2012). "Continuous aqueous two-phase extraction of human antibodies using a packed column."  880: 148-156.

Shinkazh, O., et al. (2011). "Countercurrent tangential chromatography for large‐scale protein purification."  108(3): 582-591.

Tscheliessnig, A., et al. (2014). "Ethanol precipitation for purification of recombinant antibodies."  188: 17-28.