# Thoughts on NGM for review article

Complexity

Complex is used to refer to the level of components in a system. If a problem is complex, it means that it has many components. Complexity does not evoke difficulty. On the other hand, complicated refers to a high level of difficulty.

SMB is both complex and complicated.

Nc= Number of columns using any buffers simultaneously

Nb = Number of buffer concentrates

Npat

Npumps = 3 (water, buffer, load)

Npumps = 1 (water) + 1 (load) + Nb

Valves_in: load + water + Nb

Valves_out = waste + product

# PAT

Bioreactor

| Measured quantity | Response time | Resolution | Response Range |
| --- | --- | --- | --- |
| DO, | <10 seconds | 5% points |  |
| CO2 | <10 seconds | 5% points |  |
| Titer | <10 hours | 5% points |  |
| pH (bioreactor) | <10 minutes | 0.1 | pH 6.5-7.5 |
| Glycosylation | <10 hours |  |  |
| High mannose |  | 1% point | 0-20% |
| a-fucosylation on mAbs |  | 1% point | 0-10% |
| Galctosylation on mAbs |  | 1% point | 0-80% |
| Sialation on mAbs |  | 1% point | 0-5% |

Downstream

| Measured quantity | Response time | Resolution | Response Range |
| --- | --- | --- | --- |
| pH continuous virus inactivation | <1 second | 0.05 | pH 3.0-4.0 |
| pH Anion Exchange Step | <10 seconds | 0.05 | pH 6.5-8.5 |
| pH Cation Exchange Step | <10 seconds | 0.05 | pH 4-6 |

Product concentration

| Measured quantity | Response time | Resolution | Response Range |
| --- | --- | --- | --- |

PQ

| Measured quantity | Response time | Resolution | Response Range |
| --- | --- | --- | --- |
| Aggregate/fragment or other product related homo or heterodimer impurity in load in surge tank | 10-20min | 0.5% point | 0.1-20% |
| Aggregate/fragment or other product related homo or heterodimer impurity in peak for peak cutting control | <10s | 1% point | 0.1-20% |
| Host Cell Protein in load surge tank | 10-20 min | 100ppm | 100-10,000 |
| Host Cell Protein in peak for peak cutting control | <10s | 1ppm | 1-10ppm |
| DNA in load surge tank |  |  |  |
| Protein A in load surge tank |  |  |  |