# Reliable, continuous processing using standard equipment

John Erickson

The biopharmaceutical manufacturing process and factory of 2029 should provide the following benefits:

- 6-fold reduction in lead time to expand capacity (build, qualify and make commercial product) in expansion to existing facility

  - From 3 years to 6 months (investment decision to release of commercial stock)

- >99% (300-fold) reduction in capital cost per kg/yr of BDS

  - From 500,000 $/(kg/yr) to 1,500 $/(kg/yr) which is similar to an API plant

- 3-fold inventory reduction by reducing cycle time from 18 months (12 mo + 6 mo safety stock) to 6 months with increased reliability (3 months for manufacture/release and 3 mo safety stock)

- 10-fold reduction in COGS from $100/g to < $10/g

- Reduction of product mass index (PMI), a measure of the mass of materials used for production, from 3000-20,000 kg/kg product to 100kg/kg

The long lead time for adding capacity is a significant problem because it forces companies to make investment decisions before they know how much they need, and when forecasts increase after launch, they don’t have time to build more capacity to react to the market. The high capital cost is a significant portion (usually around a third) of the COGS, which forces plant managers to try and operate at close to 100% capacity, increasing risk of stockout. Operation close to capacity is also a barrier to innovation because the plant time needed to validate and generate qualification batches of new processes eats into time available for current demand. The high capital cost also requires very high-level approvals, which delays projects or forces companies into unattractive CMO contracts. If either lead time or cost could be reduced, this wouldn’t be as much of a problem. If capital cost were high, for example, but capacity could be built in a matter of weeks, companies would only build what they knew they needed and add capacity only when there was a strong demand signal from the market. If building capacity had long lead times, but the cost was trivial, companies would overbuild just to make sure they had enough capacity. Since it is unlikely that we will be able to make lead time or cost trivial, we need to reduce both. Commercial forecasts often work on 6-month cycles, so a 6-month approval/build/validate cycle for additional capacity would fit well.

Current estimates for expanding capacity in traditional factories with traditional productivities (1-2 g/L titer in 14 days) are about $500,000 per kg/yr of capacity. In contrast, API plants cost on the order of $1,500/(kg/yr).

If we have state of the art volumetric productivity, which is about 10-fold higher than the productivity assumed for the capital estimate of $500,000/kg/yr, we could bring down the capital cost on a per kg basis by about an order of magnitude, not accounting for economy of scale. This still leaves over an order of magnitude of reduction needed to reach the target of $1,500/(kg/yr). This further reduction in CAPEX would need to come from an additional increase in cell culture productivity, but also by increasing productivity of the downstream process and by eliminating the costs associated with reinforcing floors for heavy equipment, custom equipment engineering, fabrication, installation and validation. If the equipment were small enough and therefore inexpensive enough, we could use standard sizes that were “close enough,” or even multiple pieces of standard sized equipment. Standardizing equipment would bring down the cost even more. This approach would obviously require collaboration across the industry between pharma manufacturers and equipment manufacturers. Once capital cost has been reduced, it won’t matter so much to have a facility that is completely utilized, and depreciation will no longer be a significant part of COGS. This will enable companies to build expansion space, increase security of supply and give room in the plant schedule to make further process improvements.

Inexpensive, standardized process equipment could be kept in inventory by suppliers, thus dramatically reducing lead-time. For example, a custom centrifuge has an 18-month lead-time and bioreactors and other large equipment have lead-times of a year. If new equipment could be purchased “off the shelf,” delivered in two weeks, assembled in a relatively small unclassified room, and validated in another couple weeks, the lead-time for increasing capacity could be shortened by years.

Inventory for biopharm products is high because of the high COGS and the large amount produced for validation. This is a problem because as biopharm portfolios grow, so does inventory, but the pharmaceutical industry is trying to reduce inventory to release cash for reinvestment. A single use factory can reduce inventory for new products by a factor of 3 by not making as much for validation. Further reductions in inventory can be accomplished by shortening the cycle time and reducing safety stock. Current manufacturing cycle times are on the order of 12 months and safety stock is about 6 months, meaning that a factory needs to be starting a batch 18 months before it is sold. This makes it difficult to respond to changes in market demand. A substantial improvement in reliability could enable a reduction in safety stock to 3 months. Reduction in manufacturing cycle time from 12 to 3 months would enable a total supply chain cycle time of 6 months, which is more in line with the 6-month commercial forecast cycle. This would allow multi-product plants to adjust production to the pull of the market.

In order to achieve these goals, we need a map and compass to help chose appropriate technologies and to measure progress. Capital cost estimation, though relatively straightforward, can be time-consuming and is usually practiced by a very few people in any given company, and by even fewer academics. If the industry is to work towards processes that require less capital, bench scientists and engineers need to have simple surrogates of capital cost to tell them if their ideas are going in the right direction or not. One surrogate that is commonly used by engineers is the area of the factory, which is multiplied by an average cost per square foot. For the new single-use facilities, most of the capital cost is in the building, with about 15% or so from the process equipment. The problem is that there is no obvious correlation between a process description and the floorspace it will occupy. However, floorspace is related to the footprint of the equipment, which in turn is related to the volume of the equipment. If all equipment were cube-shaped, the footprint would be proportional to the volume to the 2/3 power. More data are required to validate this model, but a study of three plants, shown in Figure 1 suggests it would be worth pursuing further.

Figure 1: Correlation between CapEx and volume

If the correlation in Figure 1 is reasonably accurate, it provides a surrogate for cost that is more accessible to people developing new technology.

Reduction in cost for a new technology can be estimated as follows.

C1C2=V1V22/3

Where C is the capital cost and V is the total process volume and subscript 1 represents legacy processes and subscript 2 represents the future state. If capital cost is to be reduced by a factor of 300 to achieve the goal set out at the beginning of this paper, the reduction in volume is described by

V1V2=3003/2=5,000

The total volume of process equipment is something that can be easily calculated from a process description and is more intuitive for people working at the bench. To normalize for the plant capacity, the important value is the volume [m3]of the equipment divided by the capacity [kg/yr]. An overall mass balance for the factory can shows the following.

θ = throughput (kg/yr)

t = batch or residence time

Vp = volume of equipment that touches the product, e.g. bioreactors, columns, holding tanks, etc.

Vb=buffer and media tank volume

C=average product concentration in process
C=θtVp

Rearranging gives

Vpθ=tC

Defining the ratio of total volume to product-contacting volume, β≡VtVp

Vtθ=βtC

Thus, the relative volume (and therefore cost) of the factory is directly proportional to the residence time and , and inversely proportional to the average product concentration. The variables were calculated for two different plants and are shown in Table 1.

Table 1: Example mass balance values

| Case | Vt (m3) | (kg/y) |  | t (days) | C (g/L) | Vt/ |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 25 | 80 | 4.5 | 7 | 0.4 | 0.3 |
| 2 | 107 | 640 | 2.5 | 7 | 0.4 | 0.2 |

A reduction in Vt/ of 5,000 yields a value of 5x10-5. I order to accomplish this,  and residence time need to minimized and average concentration needs to be maximized. Online buffer and media makeup would bring  down to unity. For example, the NIIMBL/Biophorum buffer stock skid project aims to replace buffer tanks with standard highly concentrated single component stock solutions, acid, base, and water which are mixed at the point of use. This leaves residence time and average concentrations for further volume reduction. Since mAb concentrations of 100 g/L are difficult to obtain even in formulation buffer, we assume this is the upper limit for average concentration.

Figure 2, which maps how to reach the CapEx goal set at the beginning of this paper, shows Vt/ as a function of residence time and average concentration, assuming =1. The bright red line at the upper left represents legacy processes in Table 1. The bright green line at the lower left represents the target value of 5x10-5, which is assumed to represent the target 300-fold capital reduction. This green line is theoretically possible for a downstream operation, but cells have inherent limitations. Assuming that a CHO cell has a diameter of 25µ and a specific productivity of 70 pg/cell/day, that works out to a Vt/  3x10-4 m3/(kg/yr). This means that even if the cells were packed together, with no water between them, we’d be an order of magnitude away from the 5x10-5 target. Assuming the cells were at 50mm cells/mL, which has been widely reported for continuous processes, Vt/ would be about 8x10-4. This means that higher specific productivity in CHO, or other hosts would probably be required to make the next order of magnitude reduction in volume.

Figure 2: Constraints on Reducing Vt/

Reliable continuous processing using standard parts could deliver the benefits we are looking for by reducing cycle time and increasing average concentration. If standard equipment could be put together in a standardized way, it might be possible to have a generic validation package and dramatically reduce IQ/OQ/PQ, which would gain even more time and reduce capital cost. Even further savings could be generated by using a “validation out of place” approach where modules could be validated before being swapped into the manufacturing line.

Cycle time for continuous processing is inherently shorter than for batch processing. If we co-located primary and secondary manufacturing, we could eliminate the freezing, hold, shipping and thawing steps, which would further decrease cycle time.

Continuous upstream processing for biopharmaceuticals has been done for decades, but there have been problems with process reliability, especially when keeping cultures running for long periods of time. We should put a lot of effort into technologies that will improve the reliability of continuous processes. We also need to consider the entire factory, including downstream processing, ancillary processes like media and buffer preparation, PAT, and process control. For example, as the continuous production bioreactor and chromatography columns shrink, the media and buffer tanks can grow unless technologies are developed to make media and buffers continuously from powder and eliminate the tank farms.

A NIIMBL-led program on continuous bioprocessing could achieve the goals set out above. The program would have the following component projects:

- Development of an open source, standard platform continuous bioprocess to manufacture the NIST mAb. Standard feedstocks will be made at designated laboratories and made available to technology projects. Using a standard process would enable better evaluation of improvements and facilitate collaboration between companies, universities and government. The platform will be updated as technology improvements are made.

  - The platform standard would also include standards for physical, electronic, and data connections between unit operations in continuous processing.

  - A physical process will be established, not just a design or standard. The physical system could in a NIIMBL lab, for all members to use.

- Collaboration between industry and FDA to write a case study showing what a control strategy for a continuous process could look like, along with a sample pathway for transition of a mAb from batch to the new continuous process. This case study would be analogous to the A-mAb case study of the 2000’s, but instead of establishing a common understanding of QbD, it would enable a common understanding of how to change from batch to continuous processes and develop a continuous bioprocess control strategy.

- Extension of buffer stock blending skid to media for continuous culture.

- Conceptual design of a facility in collaboration with an A&E firm, equipment suppliers, and biopharmaceutical manufacturers, that could achieve the 10-year goals above, even though some of the technology doesn’t yet exist. The design will show where innovation is needed and set goals for unit operations. This design will bring the 10-year goals to life. Previous “factory of the future” efforts have focused on new ways to build and lay out factories. This effort will be different because it focuses on the core process technologies that will be used in the factory and recognizes the link between the process and the facility that makes the process possible. This project will also confirm or refine the process volume to CapEx correlation and goals discussed above.

- Roadmap project to determine technologies required to achieve the 10-year vision. This will result in a plan for a series or projects to deliver the vision.

  - Individual projects determined by the overarching roadmap that will deliver the technologies required to achieve the 10-year vision.

    - Real-time release projects

    - Miniaturization projects

    - Reliability projects, including fault prediction and avoidance as well as making processes intrinsically more reliable through simplification.

- In collaboration with equipment suppliers, we’ll chose a unit operation for miniaturization and standardization and estimate how small such a piece of equipment would need to be, and what technology innovations would be required to make it feasible.

- Development of equipment standards required to move from custom designs to “off the shelf.”